15.5.3 Optional Lab: Test the Wireless NIC in Windows Vista

### Introduction

Print and complete this lab.

In this lab, you will check the status of your wireless connection, investigate the availability of wireless networks, and test connectivity.

### Recommended Equipment

- A computer with Windows Vista installed

- A wireless NIC installed

- An Ethernet NIC installed

- Linksys WRT300N Wireless Router

- Internet connectivity

Step 1

Disconnect the Ethernet cable from your computer.

A red “X” appears over the “Local Area Connection” icon.

Connect to a wireless network.

Hover over the “Wireless Network Connection” icon in the tray.

What is the name of the wireless connection?

Open a command window.

Ping 127.0.0.1.

How many Replies did you receive?

Use the ipconfig command.

What is the IP address of the default gateway?

Ping the default gateway.

A successful ping indicates that there is a connection between the computer and the default gateway.

Step 2

Open a web browser.

Type www.cisco.com in the “Address” field, and then press Enter.

Step 3

Open the Network Connections window.

Right-click the wireless connection and choose Connect / Disconnect.

Select All from the Show dropdown menu. Press the Refresh button.

What are the names of the wireless networks that are available?