### 14.2.4 Lab: Install an All-in-one Printer/Scanner

### Introduction

Print and complete this lab.

In this lab, you will check the Windows XP Hardware Compatibility List (HCL) for your All-in-one Printer/Scanner, install the all-in-one printer/scanner, upgrade the driver and any associated software, and test the printer and scanner.

### Recommended Equipment

- A computer with the Windows XP operating system

- An available USB port on the computer

- All-in-one Printer/Scanner

- Printer/scanner installation CD

- An Internet connection

#### Step 1

Open Internet Explorer.

Search the Microsoft Website for “windows xp hcl”.

Choose “Windows XP”.

Choose “Devices”.

Choose the processor type used by the computer.

Select “Printers & Scanners”.

Choose “Start”.

Search the HCL for your All-in-one Printer/Scanner.

What Company manufactures this component?

For what operating system(s) was this component designed?

For what operating system(s) has this component been certified?

#### Step 2

Carefully unpack and assemble the printer/scanner if necessary. Follow the manufacturer’s instructions.

Plug the printer/scanner into a grounded wall outlet and turn on the unit. Do not attach the USB cable to the computer until instructed to do so.

#### Step 3

Insert the manufacturer’s software CD into the optical drive. If the installation program does not start automatically, select Start > My Computer, and then double-click the Epson CD-ROM icon.

The “Install Menu” window appears.

Click Install.

Connect the USB cable when instructed.

The “License Agreement” window appears.

Click Yes.

The “Epson Product Registration” window appears.

Click Cancel.

#### Step 4

Go to the All-in-one Printer/Scanner manufacturer’s website and download the newest drivers for the All-in-one Printer/Scanner.

Click Start, and then right-click My Computer. Choose Manage > Device Manager.

The Device Manager appears in the right column.

Double-click the All-in-one Printer/Scanner, and then select the Driver tab.

Choose Update Driver.

Follow the instructions to install the driver that was downloaded.

Choose Start > Printers and Faxes.

Right-click All-in-one Printer/Scanner, and then click Properties.

Click the Print Test Page button.

#### Step 5

Place a picture or document in the upper left corner of the scanner, face down.

Double-click the Epson Scan icon on the desktop or open the Epson SmartPanel using the Start menu.

Scan the image.

Print the scanned image.