### 11.3.7	Worksheet: Research Computer Components

Print and complete this activity.

In this activity, you will use the Internet, a newspaper, or a local store to gather information about the components you will need to upgrade your customer’s computer. Be prepared to discuss your selections.

- Your customer already owns the case described in the table below. You will not need to research a new case.

| Brand and Model Number | Features | Cost |
| --- | --- | --- |
| Cooler Master CAC-T05-UW | ATX Mid Tower ATX, Micro ATX Compatible form factor 5 X External 5.25” drive bays 1 X External 3.5” drive bay 4 X Internal 5.25” drive bays 7 expansion slots USB, Firewire, Audio front ports |  |

- Research a power supply that is compatible with the components that your customer owns. The new component must have improved performance or additional capabilities. Enter the specifications in the table below.

| Brand and Model Number | Features | Cost |
| --- | --- | --- |
| Antec SP-450 | 450 Watt Dual +12V rails > 70% Efficiency ATX12V form factor | $60.99 |

- Research a motherboard that is compatible with the components that your customer owns. The new component must have improved performance or additional capabilities. Enter the specifications in the table below.

| Brand and Model Number | Features | Cost |
| --- | --- | --- |
| GIGABYTE GA-965P-DS3 | LGA 775 DDR2 800 PCI Express x16 SATA 3.0Gb/s interface 1.8V-2.4V RAM voltage 1066/800/533MHz Front Side Bus 4 Memory Slots Dual Channel Memory Supported 1XATA100 connector RAID 0/1 4X USB 2.0 ATX Form Factor |  |

- Research a CPU that is compatible with the components that your customer owns. The new component must have improved performance or additional capabilities. Enter the specifications in the table below.

| Brand and Model Number | Features | Cost |
| --- | --- | --- |
| Intel Core 2 Duo E6300 BX80557E6300 | LGA 775 1.86GHz Operating Frequency 1066MHz Front Side Bus 2M shared L2 Cache 64 bit supported Conroe Core | $183.00 |

- Research a heat sink/fan assembly that is compatible with the components that your customer owns. The new component must have improved performance or additional capabilities. Enter the specifications in the table below.

| Brand and Model Number | Features | Cost |
| --- | --- | --- |
| Intel Stock heat sink/fan | LGA 775 80mm fan 2 pin power Under recommended socket weight | Included with CPU |

- Research RAM that is compatible with the components that your customer owns. The new component must have improved performance or additional capabilities. Enter the specifications in the table below.

| Brand and Model Number | Features | Cost |
| --- | --- | --- |
| Patriot PDC22G6400LLK | 240-Pin DDR2 SDRAM DDR2 800 (PC2 6400) Cas Latency 4 Timing 4-4-4-12 Voltage 2.2V | $194.99 |

- Research a hard disk drive that is compatible with the components that your customer owns. The new component must have improved performance or additional capabilities. Enter the specifications in the table below.

| Brand and Model Number | Features | Cost |
| --- | --- | --- |
| Western Digital WD400BB | 40 GB 7200 RPM 2 MB Cache | $37.99 |

- Research a video adapter card that is compatible with the components that your customer owns. The new component must have improved performance or additional capabilities. Enter the specifications in the table below.

| Brand and Model Number | Features | Cost |
| --- | --- | --- |
| XFX PVT71PUDD3 | PCI Express X16 600 MHz Core clock 256 MB GDDR3 1600 MHz Memory clock 256-bit memory interface | $189.99 |

- List three components that must have the same or compatible form factor.

- List three components that must conform to the same socket type.

- List two components that must utilize the same front side bus speed.

- List three considerations when you choose memory.

- What component must be compatible with every other component of the computer?

- What determines compatibility between a motherboard and a video card?