### 6.4.1  Worksheet: Match ACPI Standards

Print and complete this worksheet.

Enter the ACPI standard next to the matching power management state description.

| S0 | S1 | S2 | S3 | S4 | S5 |
| --- | --- | --- | --- | --- | --- |

| ACPI Standard | Power Management States |
| --- | --- |
|  | The CPU is off, but the RAM is refreshed to maintain the contents. |
|  | The CPU and RAM are off. The contents of RAM have been saved to a temporary file on the hard disk. |
|  | The CPU is not executing instructions; however the CPU and RAM are still receiving power. |
|  | The CPU is off, and the RAM is set to a slow refresh rate. |
|  | The computer is off and any content that has not been saved will be lost. |
|  | The computer is on and all devices are operating at full power. |