Worksheet 5.4.2:  Preparing the Hard Drive

Match each vocabulary word with its correct definition.

Vocabulary word

1.    Primary Partition

2.    Extended Partition

3.    Active Partition

4.    Logical Drive

5.    FDISK

6.    Boot Sector

7.    Partition Table

8.    Track

9.    Sector

10.    Cluster

11.    Cylinder

12.    FAT

13.    Low-level Format

14.    High-level Format

Definition

a. This is the collective name for all tracks having the same number in a hard disk drive when several disks are stacked and rotate on a common spindle.

b. The area on each logical DOS disk where the information to boot the operating system is recorded.

c. A special file that is created and located in the disk’s sector 0 where DOS stores information about the disk’s directory structure.

d. The first partition on a DOS disk (usually contains an operating system).

e. A 512 byte “chunk” of space on a DOS disk.

f. The partition DOS refers to at boot up.

g. The process that creates a blank File Allocation Table and root directory structure on a DOS disk.

h. A physical separation of information within an extended partition.

i. The process that marks off the disk into sectors and cylinders, and defines their placement on the disk.

j. The partitioning program for MS-DOS.

k. The partition that is normally assigned all the available space left on a DOS drive.

l. Concentric circles on the disk surface created by formatting a hard drive.

m. Two or more sectors on a single track.

n. A record of the starting point of each logical drive, active partition, and master boot record.