# Cisco Discovery 1 Module 3 Quiz Picture Descriptions

# 12 Questions

1.

Refer to the description below.  What two types of devices are the printer and the IP phone? (Choose two.)

The diagram depicts an IP telephone connected to a switch.  Also connected to the switch are two computers, directly connected to pne of the computers is a standalone printer.

- The IP phone is a peripheral.

- The IP phone is a host.

- The IP phone is a network device.

- The printer is a peripheral.

- The printer is a host.

- The printer is a network device.

2.

Determine if the characteristics are an advantage of a peer-to-peer network.

List whether the statement below is an “Advantage” or a “Disadvantage”.

Advantages

Disadvantages

- Provide minimal security

- No centralized administration

- Inexpensive

- Potential impact on performance

- Easy to set-up

- No special hardware

3.

List whether the statements below are a “Logical Topology Map” or a “Physical Topology Map”.

Logical Topology Map

Physical Topology Map

- Shows wiring installations.

- Shows location of networking devices.

- Shows host names and host addresses.

- Shows group information and applications used.

- Shows location of each host and how they

- Connect to the network.

- Shows location of broadcast and collision domains.

4.

Match the situation with the correct type of message that would be used.

- Unicast

- Multicast

- Broadcast

- A host device sends a hello message to all

Computers on the network.

- A host device sends an e-mail to another

Host device.

- A host device joins a group to receive video

Conferencing.

- A host device requests a web page and

Displays the results.

- A host device sends out an ARP request

Looking for the MAC address for a specific

IP address.

- A router forwards its routing table to a specific

Group of routers on the network.

5.

Which two statements accurately describe a router ARP table and routing table? (Choose two)

- The ARP table contains information about individual devices, not networks.

- The ARP table contains information about networks, not individual devices.

- The ARP table contains information about networks and individual devices.

- The routing table contains information about individual devices, not networks.

- The routing table contains information about networks, not individual devices.

- The routing table contains information about networks and individual devices.

6.

What process can a user perform on the host so that a folder on a remote server is treated as if it were a local resource?

- mapping a drive

- sharing a drive

- enabling a remote user

- setting share permissions

7.

Considered the role of a client, server, or both.  Match the scenario wht the t role of Client, Server or Client and Server

- Client

- Server

- Client and Server

- Share files with another device.

- Download a music file from a website.

- Play a video game with a friend across a

Network.

- Participate in a videoconference with another

Computer.

- Connect to an e-learning site to learn networking

- Store email and deliver it upon request.

8.

The diagram depicts two switches named Switch A and Switch B directly connected to each other.  H1 and H2 are directly connected to switch A and H3 and H4 are directly connected to switch B.

Refer to the description above.  If H1 on Switch A needs to forward to H3 on Switch B, which MAC address is used as the destination MAC within the frame?

- MAC address of H1

- MAC address of Switch A

- MAC address of Switch B

- MAC address of H3

9.

Refer to the description below.  Two network segments, LAN-A and LAN-B, directly connect to Router 1.  When Router A receives a message with a source address of Computer 1 on LAN-A and a destination address of Computer 10 on LAN-B, what will be the action of Router 1?

The diagram depicts a Router named Router 1 and two switches directly connected to the router named Switch 1 and Switch 2. Computer 1 is located on switch 1 and computer 10 is located on switch 2.

- The router checks its ARP table to determine where to forward the packet to reach computer 10.

- The router checks it ARP table to determine the appropriate IP address of computer 10.

- The router checks its routing table to determine where to forward the packet to reach computer 10.

- The router checks its routing table to determine the appropriate MAC address of computer 10.

10.

Which three pieces of information are included in a network physical map? (Choose three)

- IP addressing scheme

- Computer naming scheme

- Location and length of cable runs

- Physical location of all networking devices

- Location and size of broadcast and collision domains

- Hardware configuration of end devices such as hosts and services.

11.

When using Windows XP, which command shows information about the computer, including IP address, sub net mask, default gateway, and additional details about DHCP and DNS?

- lpconfig

- winipcfg

- lpconfig/all

- winipcfg/all

12.

In what area of a network can traffic from other hosts cause a sending host to stop transmitting, then wait a random amount of time before resending a message?

- access layer

- broadcast domain

- collision domain

- distribution layer

- peer-to-peer network