# Module 5 - Chapter Quiz

1. A technician is setting up equipment, Which three devices will need IP addresses? (Choose three)

a printer with an Ethernet card

a web camera attached directly to a host

a server with two NIC's

an IP phone

a standalone workstation

a PDA attached to a networked workstation

2. Which three statements are true about binary octets? (Choose three)

if a bit is a zero (0), its column value is not counted.

The value of each bit in an octet is twice that of the bit to its right.

The rightmost bit in an octet has a value of 0 when it is turned on.

A host looks at every bit of an IP address as it is received by the host NIC.

If all of the bits in an octet are set to one (1), the value of the octet is 256.

There are 32 value positions in an octet.

3. Match the term to the corresponding  address Class A, B, C (Not all terms will be used)

first octet number of 0

first octet number of 126

first octet number of 130

first octet number of 200

first octet number of 224

first octet number of 240

4. Which statement is true concerning private network addresses?

They are always routed on the Internet.

They can be used by only one company at a time.

They enable Internet web servers to be easily accessed by outside users.

They are more secure as they are visible only to the local network.

5. How many Class C networks are reserved for private address space?

1

16

128

256

6. What is the destination MAC address of a broadcast Ethernet frame?

255.255.255.255

1.1.1.1

AA-AA-AA-AA-AA-AA

FF-FF-FF-FF-FF-FF

7. Match the statement to the Category (4 statements per category)

Categories

Static IP addressing

Dynamic IP Addressing

Statements

useful for printers and servers

provides more control of network resources

possibility of input errors

useful for mobile users

limited in time period

addresses provided by a server

draws addresses automatically from a pool

addresses are not reassigned to another devices

8. Place these DHCP messages in the correct order for a host to obtain an IP address from a DHCP server.

DHCP Request

DHCP Discover

DHCP Offer

DHCP Acknowledge

9. An ISR has a public IP address issued by the ISP. What is the purpose of the public IP address?

Allows internal hosts to communicate with each other on the local network

Allows internal hosts to communicate with other hosts on the Internet

allows internal the ISR to communicate with the local hosts on the network

allows the ISR to communicate with the local servers on the network

10. What function does NAT perform in an ISR?

NAT takes a source IP address and translates it to a default gateway address.

NAT takes a local IP address and translates it to an internal source IP address.

NAT takes an internal global IP address and translates it to a local source IP address.

NAT takes an internal source IP address and translates it to a global IP address.

11. Refer to the graphic, What two types of IP address should be used on the ISR at interfaces A and B? (Choose two)

Linksys WRT300N Router(R1)

R1 is connected to the ISP Router on Interface B (Public Network)

R1 is connected to the local switch on interface A (Local Network)

The Local Network has three Hosts (H1, H2, H3) which are connected via Switch

Interface A should use a private IP address.

Interface A should use a public IP address.

Interface A should use a source IP address.

Interface B should use a private IP address.

Interface B should use a public IP address.

Interface B should use a destination IP address.