# Module 6 - Chapter Quiz

1. What happens when part of a message using TCP is not delivered to the destination host?

The sender resends the entire message.

The sender sends a request to find out what happened.

Nothing. TCP does not check for errors and missing data.

The part of the message that is missing is retransmitted.

2. Match the correct statement to the Service

Service

Domain Name Services

E-mail

File Transfer

Dynamic Host Configuration

Web Service

Telnet

Statement

A user types in a URL and a page loads in the client browser.

A user uploads new files to his or her website.

A user requests a website by typing in its name.

A workstation gets its IP address automatically.

Internet protocols enable users to send messages to each other over the Internet.

A network technician logs into a workstation from a remote site.

3. Match the items with the corresponding term (Not all terms are used)

Item

DNS Server

Common DNS port number

Domain Name

Term

47

53

80

cisco.com

translates an IP address to a domain name

translates a domain name to an IP address

linksys@cisco.mail

4. Refer to the graphic, which protocol allows you to type www.cisco.com instead of an IP address to open the web page?

The picture depicts an Internet Explorer Window, www.cisco.com has been typed in the address field and the corresponding page has been loaded.

DNS

FTP

HTML

HTTP

IM

SNMP

5. Match the Term with it's corresponding definition.

Term

HTML

HTTP

TTPS

web server

web client

Definition

Hosts a web page

requests a web page

used to create web pages

secure protocol that uses port 443

protocol commonly used by a web browser

6. Match the Protocol to the corresponding category (Not all protocols are used)

Category

Protocols Used to Process E-mail

Protocols

SMTP

FTP

POP3

HTTP

IMAP4

HTML

DNS

7. What acronym is associated wit making a phone call using the Internet?

IM

HTML

HTTP

SNMP

VoIP

8. Match the Prot number to the corresponding protocol.

Port Numbers

20

23

53

68

80

110

443

Protocols

DHCP

DNS

FTP data

HTTP

HTTPS

POP3

SNMP

Telnet

9. Select three protocols that operate at the Application Layer of the OSI model. (Choose three)

ARP

TCP

DSL

FTP

POP3

DHCP