# 4 - Connecting to the Internet Through an ISP

# 4.0 Chapter Introduction

## 4.0.1 - Introduction

One Diagram

Diagram 1, Slideshow

Every day new people come online and join the human network.

From the most remote areas in Africa to metropolitan London, we need to be able to connect and communicate.

Internet Service Providers are the links to the Internet.

The web of interconnected ISPs makes the Internet

accessible to everyone.

In this chapter you will learn why ISPs are necessary and about the Network Operations Centers that are crucial to the function of the Internet.

After completion of this chapter, you should be able to:

Explain “what is the Internet?” and how we connect to the Internet using an Internet Service Provider (ISP).

Explain how information is sent across the Internet through an ISP.

Describe and identify the components of an ISP Network Operations Center.

Identify the different types of cables and connectors for connecting the devices in a Network Operations Center.

Construct and terminate twisted pair cables and determine type of cable needed.

# 4.1 – The Internet and how we Connect to it

## 4.1.1 – Explain what the Internet is.

1 Diagram

Diagram 1, Image

Explain what the Internet is.

The diagram depicts an image of the globe with links joining every continent on the planet.  It signifies that the world is connected by these virtual links as a means to share information and resources.  This network of links is now known as the Internet.  The Internet Society (ISOC) provides leadership in addressing issues that confront the future of the Internet, and is the home for the groups responsibility for Internet Infrastructure standards, including the Internet Architecture board (IAB).

Some of the main organisations that help manage and develop the Internet are:

ISOC, IAB, IETF, IRTF, IANA.

Infoplease is a good source of Internet usage statistics and resources.

http://www.infoplease.com/ipa/A0873826.html/

## 4.1.2 Internet Service Providers (ISPs)

One Diagram

Diagram 1, Relational

The picture shows graphics of several types of Internet user (Government, Home, Large Corporations, Small Businesses’, Educational Institutions) connected to an ISP (represented as a cloud). The ISP cloud connects to the Internet (also a cloud)

## 4.1.3 The ISPs Relationship with the Internet

Two Diagrams

Diagram 1, Media

The diagram shows an individual home user connected to a cloud of routers, or POPs , grouped as ISP1. A business LAN user is also connected the ISP1 cloud. Another individual Home User is connected to a similar separate cloud called ISP2. These two clouds are connected via the Internet (also represented as a cloud)

The media illustrates a message being sent from Home User 1 through the POP he is connected to, on to other POPs, through the Internet cloud to ISP2 (via several POPs) to Home User 2. Home User 2 sends a response in the same manner, but reverse order.

Diagram 2, Image

The diagram shows a map of the world with many links between cities to illustrate how POPs may be interconnected.

## 4.1.4 Options for Connecting to the ISP

Two Diagrams

Diagram 1, Relational

Diagram depicts 2 methods for connecting to the ISP. The first is a single PC with Direct connection. In this case a PC connects to a modem which in turn connects to the ISP cloud. The second method has multiple PCs connected to an integrated router. The router connects to the modem, which in tur connects to the ISP cloud.

Diagram 2, images

This diagram illustrates six devices that allow different methods of connection to the Internet.

1: Dialup modem

The slowest and most widely available service

Uses regular voice-grade telephone lines

2: Cell Modem

Service available through Cell phone providers

Relatively slow access speeds but becoming more common

3: Digital Subscriber Line (DSL)

Service available from most telephone companies

Provides high speed digital connections over regular telephone lines

4: Cable Modem

Service available from most cable companies

Provides high speed service over the cable TV network

5: Leased Lines

Service available from most telephone companies

Provides high speed connection over dedicated digital data lines

The most common example of a leased line is a T1 (E1). Primarily for a

business-class service

6: Satellite

Service available from Internet dish satellite companies

Provides medium speeds via satellite

Satellite may be the only choices faster than dial-up in some rural

areas

## 4.1.5 ISP Levels of Service

Three Diagrams

Diagram 1, Relational

Diagram shows nine services that may be offered by an ISP.

1: Content filtering

ISPs can provide software that prevents specific material from being downloaded based on user specifications.

This software is often used to block objectionable and/or offensive web sites.

2: Video on Demand

Real-time downloading of movies allows users to watch movies over the Internet. This is known as streaming video.

3: Connection Speed

Download speeds can vary from 56kbps for dialup to 1.5Mbps or higher for technologies such as DSL and cable modem.

A high speed connection is recommended for individuals who download a lot of large programs, perform gaming, or run their own servers.

4: Email Accounts

ISPs generally provide multiple user emails on one account. These email addresses can be distributed among individuals or used to separate business mail from personal mail.

Webmail allows users to access their mail from any computer connected to the Internet using any Web browser. No special/dedicated email client is required.

5: Personal Home Pages

Personal web page space is frequently provided with service. Usually both the size of the Web space and the traffic generated are limited.

Design and maintenance of the website is maintained by the individual author.

6: Web Hosing Services

Organisations that do not have web servers can use the ISPs servers for their web site. This often comes with design and maintenance services.

Web hosting services are usually purchased based on the size of the web site and anticipated monthly volume.

7: File Storage

Organisations can use the ISPs online storage and file management systems to provide 24/7 access to important files.

File storage can range from a few megabytes to terabytes of online storage.

Online file storage is usually password protected.

8: IP Telephone

ISPs may provide IP telephones services that allow users to make and receive voice calls over the Internet.

When using the Internet, regular long distance tolls do not normally apply.

9: Virus Scanning

ISPs often include virus scanning and anti-spam services as part of their connection package.

Most ISPs scan for malicious code both on the files that have been uploaded from the end user and those arriving for delivery to the end user.

Diagram 2, Image

Diagram shows the difference between Asymmetric and Symmetric connections. Asymmetric has a greater bandwidth in one direction that the other, usually a greater download bandwidth. Symmetric utilizes the same bandwidth in both directions. Main text explains further.

Diagram 3, Activity

Match each of the scenarios below with a suitable ISP.

Scenario 1: Adelle is a Network Administrator for a medium sized organization in Canada. Her company is expanding and they are planning to open a new branch in the USA. Many of the company employees work from home and access the Head Office using cable or DSL modems. They have no plans to use VoIP and do not require any email support but most of the employees are non-technical and will require support with their set-up.

Scenario 2: Maximilian is a gamer who currently attends a university. He spends all of his spare time gaming with friends or downloading music and videos. His main communication tool is Instant Messaging.

Scenario 3: Frank is a writer and travels throughout North America doing research for his novels. He stays mostly in hotels but occasionally will stay for longer periods of time with friends. He prefers DSL access if he is with friends but must be prepared to use dial-up in some of the smaller cities. He is on a very strict budget until his first novel is published.

ISP A: Low price with no set-up fee (29.95 per month) 5 email accounts Unlimited monthly access 24/7 1-800 support Established business (5 years) Cable access No DSL or VoIP support Supports Windows platform only Free child safety

ISP B: $79.98 per month (2 year contract) 30 email accounts - extra for a fee DSL and Cable access Toll-free customer support 24/7 technical support Web/POP3 email access Windows/MAC/Palm Pilot/Pocket PC support VoIP service - $30 per month 2 Gig Web space Free pop-up blockers Available in USA & Canada

ISP C: Free ISP 10 hours access per month - extra hours at $2 per hour Dial-up support Ad Banners No set-up costs Web/POP e-mail support Windows/MAC platform supported DSL support in major US cities only Free e-mail support Technical phone support at $10 per call

# 4.2 - Sending Information Across the Internet

## 4.2.1 - Importance of the Internet Protocol (IP)

Two Diagrams

Diagram 1, Image

The picture identifies the formula Hardware + Software + Connectivity = Internet.

A more Info box contains the text “RFCs are numbered documents which define protocols and other standards that determine how the Internet functions. The higher the number the newer the RFC. RFCs are submitted to the IETF and go through a review process. As they are reviewed, they go through the following stages: Proposed (entry level) Draft (initial testing) Standard (fully adopted). IETF RFC web site at: http://www.ietf.org/rfc.html”

Diagram 2, Media

The picture identifies an IP packet, and shows the packet being sent from source Host to destination Host.

## 4.2.2 - How ISP's Handle Packets

Two Diagrams

Diagram 1, Media

The picture identifies an email request being sent. The graphic contains three clouds, ISP1, the Internet and ISP2. A local user connects to ISP1 cloud. ISP1 cloud contains several POPs (routers) an email and Web server. ISP1 connects to the Internet cloud, which in turn connects to the ISP2 cloud. ISP2 cloud contains several POPs (routers) and a Web server. The email request is sent by the user to the local Router (the POP that the local user is connected to), This Router sends the email request to the NOC Router, The NOC Router sends the Request to it's Email Server, The Email Server passes the requested email back down the chain to the user.

Diagram 2, Media

This diagram is the same as for Diagram 1. The picture identifies a Request for a Web Service between two ISP's, The packet is sent from the user to the POP Router, The POP Router sends the Request to the NOC Router of ISP1, NOC Router of ISP1 sends the request to the local Router of ISP2, The local Router of ISP2 sends the request to the NOC Router of ISP2, The NOC Router of ISP2 sends the request to the Web Server, The Web server passes the requested information back down the chain to the user.

## 4.2.3 - Forwarding Packets Across the Internet

One Diagram

Diagram 1, Image

The Diagram depicts several routers within the Internet cloud. The users connect to the York router, York connects to London router, London to Paris and Paris to Rome.  The picture identifies how a packet travels over the Internet from York to Rome using traceroute.

Starting at York

London - 209.165.201.1, 255.255.255.224

Rome - 209.165.200.225, 255.255.255.224

Paris - 209.165.202.129, 255.255.255.224

Output of the traceroute command is

York#traceroute ROME

Type escape to abort.

Tracing the route to ROME (209.165.200.225)

LONDON (209.165.201.1) 8msec 8msec 4msec

PARIS (209.165.202.129) 8msec 8msec 8msec

ROME (209.165.200.225) 8msec 8msec 4msec

There are 2 following pages with links to a packet tracer exersice and a Hands on Lab.

# 4.3 - Networking Devices in a NOC

## 4.3.1 - Internet Cloud

One Diagram

Diagram 1, Media

The picture identifies how information is sent over the Internet from source to destination

1. Source sends packet

2. Packet travels through multiple routers within the cloud (Internet)

3. Packet reaches destination

## 4.3.2 - Devices in Internet Cloud

Three Diagrams

Diagram 1, Image

Diagram depicts two methods of connection described in the text body. The network consists of two Businesses (Business1, Business2) and two Homes (Home1, Home2)

Business1 and Home 1both have a PC connected to an Integrated Router which connects to a Cable Modem. The Cable modem connects to the cable TV system, which in turn is connected to the cable providers CMTS

Business2 and Home 2 both have a PC connected to an Integrated Router which connects to a DSL Modem. The DSL modems connect via the voice phone system to the ISPs DSLAM.

Diagram 2, Image

The picture identifies different equipment, which is used in Home or Small Business Networks including Linksys WRT300N Router, Cisco 1800 series ISR, Cisco 7204 - VXR Router, Cisco 6513 Chassis Bundle.

Diagram 3, Activity

The following is an Interactive Media Activity, which identifies different equipment used by an ISP.

Amelia has been hired as a Network Support Analyst for a major ISP. On her first day, she will be going on a tour of the NOC's equipment room.

Amelia is expecting to see certain devices that support required features. She has started to compile a list of these required features. Match the requirement with the device that will support it.

Requirement 1: ISPs handle extremely large volumes of traffic very quickly. Requirement 2: ISPs need to function at near 100% uptime since the failure of a key piece of equipment at an ISP can have disastrous effects on network traffic.

Requirement 3: The ISP must store and support the many services that they provide their customers, such as DHCP, email, web access.

Requirement 4: The ISP has users using DSL technology.

Requirement 5: The ISP has users using cable modems to connect.

Device A: Servers

Device B: DSLAM

Device C: Backup Equipment

Device D: CMTS

Device E: High Speed Devices

## 4.3.3 - Physical and Environmental Requirements

Three Diagrams

Diagram 1, Image

The picture identifies devices including a Linksys WRT300N Router and Networking Cabinets.

Diagram 2, Image

The picture identifies Uninterrupted Power Supplies (UPS) that can be used in both the Home and small Business and Enterprise.

Diagram 3, Image

The picture identifies a technician working on Switch cabinets.

# 4.4 – Cables and Connectors

## 4.4.1 – Cables and Connectors

2 Diagrams

Diagram 1, Image

Common Network Cables

The diagram depicts a network cloud with the Source PC at the entry point into the cloud.  The source is directly connected to a switch and then to a boundary router which acts as the gateway to the Internet or cloud.  Housed within the cloud are 7 routers and there are 3 other boundary routers connecting ambiguous networks to the Internet.  Connected to one of the networks is the destination PC on the far side of the cloud which defines that the destination PC may be on the other side of the world.  The connections between source and destination may include multiple different types of cabling or media to transmit on.  The data leaves the source on a straight through cable to the switch. The switch forwards the data by another straight through cable to the boundary router.  These connections are copper and within the cloud the data is transmitted by serial copper links (electrical) or fibre optic (light) or even wireless transmission (RF).  When the data reaches the far side boundary router it is converted back to an electrical signal and forward by a straight through cable from the router to the switch and another straight through cable between the switch and the destination PC.  The key point outlined by this image is that several types of cabling media are used between source and destination.

Diagram 2, Image

Common Network Cables

The diagram depicts several different types of cabling media used in the transmission of data across the network.  Listed in the image are Twisted Pair, Coaxial cable and Fibre optic cabling.  The information pertaining to each of the mediums is as follows:

Twisted Pair:

TP is one of the most commonly used cable types in networking.  Wires are grouped in pairs and twisted together to reduce interference.  The pairs of wires coloured so that you can identify the same wire at each end.  Typically in each pair, one of the wires is a solid colour and its partner is the same colour striped onto a white background.

Coaxial Cable

Coaxial was one of the earliest types of network cabling developed.  Coaxial cable is the kind of copper cable used be cable TV companies.  Coaxial cable has a single rigid copper core that conducts the signal.  This core is typically surrounded by a layer of insulation, braided metal shielding and protective jacket.  It is used as a high-frequency transmission line to carry a high frequency or broadband signal.

Fibre Optic

Fibre optic cable or can be made of glass or plastic with a diameter about the same as a human hair that carry digital information at very high speeds over long distances.  Since they use light instead of electricity, electrical interference does not affect the signal.  Fibre optic cables have many uses as well as communications.  They are also used in medical imaging, medical treatment and mechanical engineering inspection.

## 4.4.2 – Twisted Pair Cables

2 Diagrams

Diagram 1, Media

Twisted Pair Cables

The diagram depicts 4 different graphs each with time along the x axis and voltage along the y axis.  The first graph depicts a Pure Digital Signal and this can be represented as ON or OFF, 0 or 1. In this example a 0 is represented by 0 volts and a 1 by 3 volts.

Plotted on the second graph is the Interference Signal which is depicted by a sharp sudden rise and drop in the signal with no uniformity.

Plotted on the third graph is the Digital Signal with Interference which is depicted as the digital signal overlaid with the Interference signal.  (ie the 2 signals are added together) To visualize this, first think of the a digital or binary signal which is blocked as ON or OFF.  Then think of the trough and the peak of this signal and imagine there is no stringent ON/OFF sequence and that there is no defined 0 or 1 status.  This interference signal can therefore be defined as a corrupted signal due to the fact there is no clear 0/1 translation.

Plotted on the fourth graph is the What the Computer Reads model which is depicted as an analogue signal with lows (troughs) and highs (peaks) in the signal which are representing the 0 and 1 translation within a digital signal. Due to this interference, the received signal is incorrectly interpreted at the receiver.

Diagram 2, Image

Twisted Pair Cables

The diagram depicts the different types pf Twisted Pair cables that are commonly found in networking applications currently, there characteristics are listed below:

Category 3 UTP cable

- used for voice communication

- most often used for phoneline

Category 7 Cable (ScTP)

- used for data transmission

- individual pairs are wrapped in a shield and then the entire four pairs are wrapped in another shield

- Supports 1000Mbps and 10Gbps, though 10Gbps is not recommended.

Category 6 UTP Cable

- used for data transmission

- an added separator is between each pair of wires allowing it to function at higher speeds

- Supports 1000Mbps – 10Gbps although 10Gbps is not recommended.

Category 5 UTP Cable

- used for data transmission

- Cat 5 supports 100Mbps and can support 100Mbps but is not recommended

- Cat 5e supports 1000Mbps

## 4.4.3 – Coaxial Cable

1 Diagram

Diagram 1, Image

Coaxial Cable

The diagram depicts an image of a coaxial cable that has been broken down into pieces so that a better understanding of its physical composition can be attained, the breakdown is as follows:

TERMINATION

- coaxial is usually terminated with a BNC or F-series connector BNC is a crimped down connector and is typically considered a stronger connection

- F-series connectors is a screw down connector

BRAID

- a metallic braid of aluminum or foil helps to shield against EMI

INSULATOR

- an insulator, usually made of steel, that both protects against EMI and provides strength to the cable allowing it to be more pliable.

CONDUCTOR

- a single centre conductor, most often made of copper, though aluminum can be used as well.

## 4.4.4 – Fibre Optic Cables

3 Diagrams

Diagram 1, Image

Fiber Optic Cable

The diagram depicts a fiber optic cable that has been dissected into separate parts for greater understanding of its physical components.  The connectors at either end of the cable have separate Rx and Tx lines for transmission to and from source to destination.  The composition of the fiber cable is as follows:

JACKET

- added to protect the fiber against abrasion, solvents and other contaminants.  This outer jacket composition can vary depending on this cable usage.

STRENGTHENING MATERIAL

- surrounds the buffer, prevents the fiber cable from being stretched when it si being pulled.  The material used is often the same material used to produce bulletproof vests.

BUFFER

- used to help shield the core and cladding from damage.

CLADDING

- made from slightly different chemicals than those used to create the core.  It tends to act like a mirror reflecting light back into the core or the fiber.  This keeps light in the core as it travels down the fiber.

CORE

- the core is actually the light transmission element at the centre of the optical fiber.  This core is typically silica, or glass.  Light pulses traveling through the firer core.

Diagram 2, Image

Fiber Optic Cable

The diagram depicts the two different types of fiber optic.  The two types are Single Mode and Multimode Fiber optic.  The characteristics that pertain to these fiber cables are as follows:

SINGLE MODE

- Polymeric coating

- Glass cladding is 125 microns in diameter

- Glass ocre is 9 microns in diameter

- Produces single straight path for light

- Small core

- Less dispersion

- Suited for long distance applications

- Uses lasers as the light source

- Commonly used for campus backbones for distances of several thousand meters

MULTIMODE

- Standard PVC coating

- Glass cladding is 125 microns in diameter

- Glass core is between 50 – 62.5 microns in diameter

- Allows for multiple path for light to travel

- Larger core than single mode cable

- Allows greater dispersion and therefore loss of signal

- Suitable for long distance applications, but shorter than single mode cable

- Uses LED’s as the light source

- Commonly used with LAN’s or distances of a few hundred meters within a campus network.

Diagram 3, Activity

Fiber Optic Cable

The diagram depicts an activity where you must decide if fiber is the best solution to a cabling requirement. After each scenario, select whether it requires Fiber Optic or Copper.  The scenario’s are listed below:

- A company must provide network connectivity between three buildings on a single campus.  The cables must be run outside and there is a high probability of lightning storms in the area.

- A company must provide network connectivity between two buildings located 1 KM apart.

- A company must provide 100Mbps connectivity to users located in their main office by running cables from the central switch to the individual desktops.  The maximum distance from the switch to the workstation is 60 meters.

# 4.5 - Working with Twisted Pair Cabling

## 4.5.1 - Cabling Standards

The picture identifies different cabling standards and where they are used in the world.

North America - Canadian Standards Association (CSA)

North America - IEEE

North America - ANSI / TIA / EIA

South America - ANSI / TIA / EIA (de facto)

Europe - ITU (International Telecommunication Union)

Europe - British Standards

Europe - CEN/CENELEC

Asia - National Quality Technology Bureau and Construction Department GB/T

Asia - Japanese Standards Association

Australia - AS / NZS (ACA)

## 4.5.2 - UTP Cables

Four Diagrams

Diagram 1, Image

The picture identifies how pairs 2, 3 are swapped in the following cable

Diagram.

T568A

1 - Green/White, Pair 3

2 - Green, Pair 3

3 - Orange/White, Pair 2

4 - Blue

5 - Blue/White

6 - Orange, Pair 2

7 - Brown/White. Pair 4

8 - Brown, Pair 4

T586B

1 - Orange/White, Pair 2

2 - Orange, Pair 2

3 - Green/White, Pair 3

4 - Blue

5 - Blue/White

6 - Green, Pair 3

7 - Brown/White, Pair 4

8 - Brown, Pair 4

Diagram 2, Image

The picture identifies a UTP cable, which has had the outer shield stripped back to expose the wires.

Diagram 3, Image

The picture identifies a TIA/EIA Straight Through cable

TIA/EIA straight through cabling

1 - Green/White, Transmit

2 - Green, Transmit

3 - Orange/White, Receive

4 - Blue

5 - Blue/White

6 - Orange, Receive

7 - Brown/White

8 - Brown

1 - Green/White, Receive

2 - Green, Receive

3 - Orange/White, Transmit

4 - Blue

5 - Blue/White

6 - Orange, Transmit

7 - Brown/White

8 - Brown

Diagram 4, Image

The picture identifies a TIA/EIA Cross-Over cable, and how the Receiving and transmitting wire pairs are swapped at either end of the cable.

TIA/EIA Cross-Over cable

1 - Orange/White, Transmit

2 - Orange, Transmit

3 - Green/White, Receive

4 - Blue

5 - Blue/White

6 - Green, Receive

7 - Brown/White

8 - Brown

1 - Green/White, Receive

2 - Green, Receive

3 - Orange/White, Transmit

4 - Blue

5 - Blue/White

6 - Orange, Transmit

7 - Brown/White

8 - Brown

## 4.5.3 - UTP Cable Termination

One Diagram

Diagram 1, Image

The picture identifies the wire grouping for one side of a UTP cable, and the connector, which the wires slid into.

## 4.5.4 - Terminating UTP at Patch Panels and Wall Jacks

Diagram 1, Image

The picture identifies a Patch Panel (Front and Rear), and a punchdown tool.

Diagram 2, Image

The picture identifies a wall jack (Front and Rear)

Diagram 3, Video

The picture identifies how to connect wires from a UTP cable to a RJ45 wall jack.

## 4.5.5 - Cable Testing

Three Diagrams

Diagram 1, Image

The picture identifies a Cable Tester, Cable  Certifier, Multimeter, and gives a brief description of each.

Cable Tester

Cable testers are used to test cables for various wiring faults such as a wires accidentally mapping to the wrong pin, or shorts or opens within the cable.

Cable Certifier

A cable certification tester determines the precise performance of a cable, and then records it in graphical form for the customer records.

Multimeter

Multimeters measure AC/DC voltage, electric current, and other cable and electrical characteristics.

Diagram 2, Image

Reversed Pair

The reversed-pair fault occurs when a wire pair is correctly installed on one connector, but reversed on the other connector. For example if the white/green wire is terminated on pin 1 and the green wire is terminated on pin 2 at one end of a cable, but reversed at the other end, then the cable has a reversed-pair fault.

Split Pair

A split-pair fault occurs when one wire from one pair is switched with one wire from a different pair at both ends. Look carefully at the pin numbers in the graphic to detect the wiring fault. A split pair creates two transmit or receive pairs each with two wires that are not twisted together. This mixing hampers the cross-cancellation process and makes the cable more susceptible to crosstalk and interference.

Open

An error in wiring that is caused by a break in the continuity of a circuit.

Short

A short occurs when the copper portions of two wires touch each other.

Diagram 3, Image

The picture identifies attenuation, Two Hosts are connected to a hub, HostA sends a message to HostB, When the Hub receives the message, it amplifies it and retransmits the packet on towards HostB. However, when HostB receives the message it is too faint to read due to the attenuation.

## 4.5.6 - Cabling Best Practices

Three Diagrams

Diagram 1, Image

The picture identifies a technician working on networking equipment

Diagram 2, Image

The picture identifies a technician working on networking equipment

Diagram 3, Image

The following is an Interactive Media Activity, which identifies cabling practices. Answer yes or no if an operation is a cabling best practice.

Question 1

In an effort to prevent people from tripping over cables, Charles ran the cables through the false ceiling over fluorescent lights.

Question 2

Yasmin labeled all cables and recorded their location as she installed them.

Question 3

Damon decided to follow T568A termination cabling standard for all straight-through cabling needs in his network.

Question 4

Samuru needed to connect a device to the network.  She located a bunch of old, hand-terminated cables in a drawer.  Assuming the cables were good, she selected one and connected her device.

# 4.6 - Chapter Summary

## 4.6.1 - Summary

One Diagram

Diagram 1, Tabular

This chapter described the Internet as a worldwide collection of computer networks, cooperating with each other to exchange information using common standards. In order to access the Internet you must connect through an ISP.

An ISP is a company that provides the connections and support to access the Internet.

ISPs support small to large geographic areas.

ISPs provide services such as e-mail, web pages, IP Telephony and DNS.

Individual computers and local networks connect to the ISP at a Point of Presence

Connection to the ISP can be done via dial-up, DSL, cable, wireless or satellite.

The most important protocol of the Internet is the Internet Protocol (IP).

The Internet Protocol formats data into packets that are from 500 to 1500 bytes in length and that must contain a valid source and destination IP address.

IP addresses must be unique on the Internet.

Every ISP has a control facility for their network, known as the Network Operations Center (NOC) which usually controls traffic flow and houses services such as email and web hosting.

Ping and tracert are utilities that test connectivity to a destination device.

On network diagrams, a cloud is often used to represent the Internet without showing the details of the connections.

In the Internet cloud, routers are used to provide alternate paths from source to destination.

ISPs must be able to accept and deliver information to end users and other ISPs using a variety of technologies.

Networking devices used in ISPs are high-end, high-speed devices with redundancy.

An ISP must consider the physical requirements, such as power and air conditioning, of the networking equipment, and the servers it houses.

For communication to occur a channel must be present to carry the signals. In the networked world, the medium is usually a physical cable:

Network cables are grouped into two categories: copper cables that carry electrical signals and fiber cables which carry light waves.

Copper wire, such as UTP and coaxial cable, is extremely vulnerable to both electro-magnetic (EMI) and radio frequency interference (RFI).

Fiber optic cable is made of glass or plastic and is immune to EMI and RFI.

Unshielded Twisted Pair (UTP) cable is used on most Ethernet networks.

Coaxial cable, used frequently for cable TV and Internet, has an extremely large bandwidth capacity which allows many signals to be combined or multiplexed together.

Fiber optic cable can provide a tremendous amount of bandwidth and is suitable for use as backbone cables.

Cabling standards are guidelines for the installation and testing of cables to ensure acceptable performance as technology evolves.

TIA/EIA twisted pair termination standards are 568A and 568B.

Different types of cables have different wiring schemes.

Common UTP cable types are straight-through, crossover, and rollover.

Cable testers check for continuity, open terminations, shorts, split pairs and reversed pairs. A cable meter tests for attenuation and crosstalk.

Cabling best practices help to reduce the number of cable errors and problems that can be encountered and can save a great amount of time, money and frustration.