CCNA Discovery

Networking for Home and Small Businesses

Lab 3.3.3 Determine the MAC Address of a Host

Objective

- Determine the MAC address of a Windows XP computer on an Ethernet network using the ipconfig /all command.

- Access to the Run command.

Background/Preparation

Every computer on an Ethernet local network has a Media Access Control (MAC) address that is burned into the Network Interface Card (NIC). Computer MAC addresses are usually displayed as 6 sets of two hexadecimal numbers separated by dashes or colons. (example: 15-EF-A3-45-9B-57). The ipconfig /all command displays the computer MAC address. You may work individually or in teams.

The following resources are required:

- Windows XP workstation with at least one Ethernet network interface card (NIC)

Step 1: Open a Windows command prompt window

- From the Windows XP desktop, click Start then Run.

- Type cmd in the Run dialogue box then click OK.

- A Windows command prompt window opens.

Step 2: Use the ipconfig /all command

- Enter the ipconfig /all command at the command prompt.

- Press Enter. (Typical results are shown in the following figure, but your computer will display different information.)

Step 3: Locate the MAC (physical) address(es) in the output from the ipconfig /all command

- Use the table below to fill in the description of the Ethernet adapter and the Physical (MAC) Address:

| Description | Physical Address |
| --- | --- |

Step 4: Reflection

- Why might a computer have more than one MAC address?

____________________________________________________________________________

____________________________________________________________________________

- The sample output from the ipconfig /all command shown previously had only one MAC address. Suppose the output was from a computer that also had wireless Ethernet capability. How might the output change?

____________________________________________________________________________

____________________________________________________________________________

- Try disconnecting the cable(s) to your network adapter(s) and use the ipconfig /all command again. What changes do you see? Does the MAC address still display? Will the MAC address ever change?

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

- What are other names for the MAC address?

____________________________________________________________________________

____________________________________________________________________________