- When configuring an ISR device using Cisco SDM Express Wizard, what does setting the Enable Secret Password field accomplish?

  - ensures that authorization must be granted before accessing the Internet

  - blocks unauthorized users from accessing the LAN

  - controls access to user executable mode

  - controls access to privileged mode

- When using Cisco SDM, which WAN encapsulation type can be configured to require a username and password before a connection is granted?

  - high-level data link control (HDLC)

  - frame relay

  - point-to-point protocol (PPP)

  - ATM PVC

- What speed and duplex setting will result on a Catalyst switch if it is set to auto-negotiate speed and duplex and is connected to a 100 Mbps port on a device that does not support auto-negotiation?

  - 10 half duplex

  - 10 full duplex

  - 100 half duplex

  - 100 full duplex

- Which method can be used to configure a Cisco Catalyst switch before an IP address has been applied to the management interface?

  - Cisco IOS CLI using Vlan 1

  - Cisco IOS CLI using console port

  - Cisco device manager using console port

  - CiscoView software using Vlan 1

- What is a secure way that a client can connect to a device in-band for the purpose of remote monitoring and administration?

  - Telnet

  - HTTP

  - SSH

  - console port

- Which type of wide area network (WAN) connection uses packet switched networks?

  - ISDN

  - dial-up

  - frame relay

  - point-to-point

- A small company with two offices in the same building is requesting advice on WAN connections.  Which two questions would give a technician information to base a recommendation?  (Choose two.)

  - What operating system is being used?

  - How much money has the customer budgeted to spend on the WAN connection?

  - What type of e-mail client software is used by the employees?

  - Are the computers laptops or workstations?

  - Are the company web servers located in the building or at the ISP?

- What is one fundamental difference between Cisco's CLI versus the SDM interface?

  - The SDM interface can be used with both in-band and out-of-band management.

  - The CLI interface can be used with both in-band and out-of-band management.

  - The SDM interface requires a terminal emulation program on the PC.

  - The CLI interface cannot be used over a Telnet connection.

- Which two statements describe the command history feature?  (Choose two.)

  - It requires configuration of a history buffer before it can be used.

  - It displays the most recently entered command strings in the current mode.

  - It saves the output from the most recent show commands.

  - It displays the last five commands that were entered in global configuration mode.

- It can be accessed by using the up and down arrow keys.

- Which router mode displays a prompt of Router#?

  - global configuration mode

  - privileged EXEC mode

  - setup mode

  - user EXEC mode

- In which two cases would out-of-band management of a router be required? (Choose two.)

  - when accessing a customer router from the ISP to monitor the normal operation

  - to access and configure the router before the IP network is operational

  - to correct an error that has shutdown the network interfaces on a router

  - when the NAT translation configuration settings are incorrect

  - to back up the running configuration on a tftp server

- Which two statements describe the result of entering the ip route 0.0.0.0 0.0.0.0 192.168.1.1 command on a router? (Choose two.)

  - The router is not able to reach the 192.168.1.0 network.

  - All packets received by the router are sent to the address 192.168.1.1.

  - The remote network 192.168.1.0 can be reached using any interface.

  - A default static route is added to the routing table.

  - If a route to a destination network is not known, the packet is sent to 192.168.1.1.

- Match the command on the top to the correct category on the bottom

  - Enable

  - ip address 172.16.1.1 255.255.255.0

  - show ip route

  - ping

  - no shutdown

  - configure terminal

  - show interfaces

  - interface fastethernet 0/0

  - Used to change router modes or sub-modes

  - Used by administrator to verify or monitor router operation

  - Affects the operation of the network

- What is the purpose of assigning an IP address to the interface VLAN1 on the Cisco switch?

  - to be able to telnet to the switch to manage and configure it

  - to enable the switch to route between networks

  - to create a new IP local network on the switch

  - to permit IP packets to be forwarded by the switch