Lab 4.2.4 Determining PAT Translations

1	2

Request

Client

Translated Response

Translated Request

NAT Router

4

One global IP address

Response

Web Server

3

1	Client on a private network sends a request to a web server on the public Internet.

2	NAT router translates source address and forwards the request to the web server

3	The web server responds to the client’s translated address

4	The NAT router translates the client address (destination) back to the original private address

Presentation_ID	©2006 Cisco Systems, Inc. All rights reserved.	Cisco Confidential	1

Objectives

- Explain the active network connections open on a computer when viewing a particular web page.

- Determine what an internal IP address and port number are translated to using port address translation (PAT).

Background / Preparation

Port address translation (PAT) is a form of network address translation (NAT). With PAT, the router translates multiple internal (usually private) addresses to a single public IP address on an interface that is connected to the Internet. Port numbers are used, in combination with IP addresses, to keep track of individual connections. In this lab, you use the ipconfig and netstat commands to view open ports on a computer. You will be able to see the initial IP address and port combination, and determine the translated IP address and port combination.

The following resources are required:

- Computer running Windows XP Professional

- Connection to a gateway router or an ISR using PAT

- Internet connection

- Access to the PC command prompt.

Step 1: Determine the IP address of the computer

a. Open a Command Prompt window by clicking Start > Run and typing cmd. Alternatively, you may click Start > All programs > Accessories > Command Prompt. At the prompt, type the ipconfig command to display the I P address of the computer.

- What is the IP address of the computer?

- Is there a port number shown, and why or why not?

Step 2: Determine the IP addresses of the gateway router or ISR

Check with your instructor to get the IP addresses for the ISR NAT router gateway. Internal Ethernet address:	
External Internet address:

Step 3: Display baseline netstat results

- At the command prompt, type the netstat –n command.

- What type of information does the netstat –n command return?

- Where does the IP address found in Step 1 appear? Is there a port number associated with it? Why or why not?

Step 4: Display active network connections

- Ping www.cisco.com and record the address.

- Open a web browser and enter www.cisco.com in the address bar.

- Go back to the Command Prompt window. Type the netstat –n command again, and then type the command without the –n option. The output looks similar to the following figure, depending on what other network applications and connections are open when you issued the command.

- What is the difference in the output between the netstat and netstat –n commands?

- Write down the connection entries for the client IP address and the IP address of the www.cisco.com web server.

Local client IP address and port number:

Foreign IP Address and port number:

- Are there more netstat entries the second time?

Step 5: Determine translated addresses

Use the information recorded in steps 2 and 4 and the topology diagram shown at the beginning of the lab to fill in the Address: Port columns.

Presentation_ID	© 2006 Cisco Systems, Inc. All rights reserved.	Cisco Confidential	2

Request

|  | Type | Address:Port |
| --- | --- | --- |
| Source | Inside-Local |  |
| Destination | Outside-Local |  |

Translated Request

|  | Type | Address:Port |
| --- | --- | --- |
| Source | Inside-Global |  |
| Destination | Outside-Global |  |

1	2

NAT Router

4

Translated Response

|  | Type | Address:Port |
| --- | --- | --- |
| Source | Outside-Local |  |
| Destination | Inside-Local |  |

ISP

3

Response

|  | Type | Address:Port |
| --- | --- | --- |
| Source | Outside-Global |  |
| Destination | Inside-Global |  |

Web Server

Step 6: Reflection

- Port address translation (PAT) is also called NAT with overload. What does the term “overload” refer to?

- The NAT terminology used in the lab includes four types of addresses: inside-local, inside-global, outside-local, and outside-global. In many connections that pass through NAT routers, two of these addresses are often the same. Which two of these four addresses normally remain unchanged, and why do you think that is the case?