| CCNA Discovery Working at a Small-to-Medium Business or ISP |  |
| --- | --- |

Lab 5.3.7 Configuring DHCP with SDM and the Cisco IOS CLI

| Device | Host Name | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- | --- |
| R1 | Customer | Serial 0/0/1 (DTE) | 209.165.200.225 | 255.255.255.224 |
|  |  | Fast Ethernet 0/0 | 192.168.1.1 | 255.255.255.0 |
| R2 | ISP | Serial 0/0/0 (DCE) | 209.165.200.226 | 255.255.255.224 |

Objectives

- Configure a customer router for DHCP using SDM.

- Configure a customer router for DHCP using the Cisco IOS CLI.

- Configure a DHCP client.

- Verify DHCP functionality.

Background / Preparation

In this lab, you set up a customer router to act as a DHCP server for internal client computers. DHCP assigns an address, subnet mask, and default gateway to hosts dynamically from a defined pool of addresses.

Set up a network similar to the one shown in the topology diagram. Any router that meets the interface requirements displayed in that diagram – such as 800, 1600, 1700, 1800, 2500, and 2600 routers, or a

combination – may be used. Refer to the Router Interface Summary table at the end of the lab to determine which interface identifiers to be used based on the equipment in the lab. Depending on the router model, output may vary somewhat from that shown in this lab.

Required Resources

The following resources are required:

- Cisco 1841 ISR router (or comparable) with SDM version 2.4 or above installed to act as the customer router

- Cisco 1841 router (or other router) to act as the ISP router

- Cisco 2960 switch (or other switch/hub) to connect hosts H1, H2, and the customer router

- Windows XP computer (host H1) with Internet Explorer 5.5 or later and Sun Java Runtime Environment (JRE) version 1 .4.2_05 or later (or Java Virtual Machine (JVM) 5.0.0.3810)

- Windows XP computer (host H2)

- Straight-through Category 5 Ethernet cables

- Null serial cable (R1 to R2)

- Console cables (H1 to R1 and H2 to R2)

- Access to the host H1 and H2 command prompt

- Access to the host H1 and H2 network TCP/IP configuration
From hosts H1 and H2, start a HyperTerminal session with each router.

Note: Make sure that the routers and the switch have been erased and have no startup configurations. Instructions for erasing are provided in the Lab Manual, located on Academy Connection in the Tools section. Check with the instructor if you are unsure of how to do this.

Task 1: Configure Basic Router Settings

Step 1: Build the network and configure host computer IP settings.

- Make sure that the host computers are connected according to the topology diagram.

Note: A router other than the 1841 may require a connection to a port other than Fast Ethernet 0/0 to access SDM.

- Configure host H1 with the following static IP information.

IP address: 192.168.1.101 Subnet mask: 255.255.255.0 Default gateway: 192.168.1.1

- Configure host H2 as a DHCP client. Choose Start > Settings > Control Panel > Network Connections > Local Area Connection. Click the Properties button and then Internet Protocol (TCP/IP) Properties. Select the options Obtain an IP address automatically and Obtain a DNS server address automatically.

- On hosts H1 and H2, open a command prompt. Click Start > Run, and then type cmd and press Enter. Alternatively, choose Start > All Programs > Accessories > Command Prompt. Issue the ipconfig /all command. Record the MAC addresses for H1 and H2.

Host H1 MAC address Host H2 MAC address

Step 2: Configure the customer router basic settings with the Cisco IOS CLI.

Configure the host name, passwords, interfaces, and HTTP service in preparation for the use of SDM. Also configure a default route to the ISP.

Router>enable

Router#config t

Router (config) #hostname Customer

Customer (config) #enable secret class

Customer (config) #username admin privilege 15 secret cisco123

Customer (config) #no ip domain-lookup Customer (config) #line con 0

Customer (config-line) #password cisco Customer (config-line) #logging synchronous Customer (config-line) #login

Customer (config-line) #line vty 0 4 Customer (config-line) #password cisco Customer (config-line) #login

Customer (config-line) #exit

Customer (config) #interface FastEthernet0/0

Customer (config-if) #description LAN Default Gateway

Customer (config-if)#ip address 192.168.1.1 255.255.255.0

Customer (config-if) #no shutdown

Customer (config-if) #interface Serial0/0/1

Customer (config-if)#ip address 209.165.200.225 255.255.255.224

Customer (config-if)#description WAN link to ISP

Customer (config-if) #no shutdown Customer (config-if) #exit

Customer (config) #ip http server

Customer (config) #ip http authentication local

Customer (config) #ip route 0.0.0.0 0.0.0.0 209.165.200.226

Step 3: Configure ISP router basic settings with the Cisco IOS CLI. Configure the host name, passwords, and interfaces.

Router>enable

Router#configure terminal

Router (config) #hostname ISP

ISP (config) #enable secret class ISP (config) #line console 0

ISP (config-line) #password cisco ISP (config-line) #login

ISP (config) #line vty 0 4

ISP (config-line) #password cisco ISP (config-line) #login

ISP (config-line) #exit

ISP (config) #exit

ISP (config) #interface serial 0/0/0

ISP (config-if)#description WAN link to Customer

ISP (config-if)#ip address 209.165.200.226 255.255.255.224 ISP(config-if)#clock rate 64000

ISP (config-if) #no shutdown ISP (config-if) #exit

ISP(config)#ip http server ISP (config) #exit

Step 4: Save the router configurations.

From privileged EXEC mode, save the running configuration to the startup configuration.

Cus tomer#copy running-config startup-config I SP#copy running-config startup-config

Step 5: Connect to Customer with host H1 using SDM.

a. On H1, disable any popup blocker programs. Popup blockers prevent SDM windows from displaying.

- The SDM GUI does not load automatically on the router. You must open a web browser to access SDM. Go to http://1 92.168.1.1. (The I P address of the Customer FastEthernet 0/0 interface – the H1 default gateway)

Note: If the browser cannot connect, check the cabling and connections and make sure that the PC IP configuration is correct.

- In the Connect to dialog box, enter admin for the username, and cisco123 for the password. The login information was configured in Step 2. Click OK. The main SDM web application starts. If you are prompted to use HTTPS, click Cancel. If a Security Warning window displays, click Yes to trust the Cisco application.

- Verify that you are using SDM 2.4 or later. The initial SDM screen that displays immediately after the login shows the current version number. It is also displayed on the main SDM screen as shown below, along with the Cisco IOS version.

Note: If the version is not 2.4 or later, notify the instructor before continuing with this lab. You must download the latest zip file from the SDM web page and save it to the PC. From the Tools menu of the SDM GUI, choose Update SDM to specify the location of the zip file and install the update.

Step 6: Configure SDM to show the Cisco IOS CLI commands.

- From the Edit menu in the main SDM window, choose Preferences.

- Check the Preview commands before delivering to router box. When this option is checked, you can view the Cisco IOS CLI configuration commands before they are sent to the router, which is a good way to learn about the commands used.

Step 7: Select additional tasks from the Configure menu.

- Click the Configure button at the top of the SDM window and select Additional Tasks from the Task menu at the left of the screen. In the Additional Tasks menu, click the plus sign (+) next to DHCP to expand the menu, and then click DHCP Pools.

- In the DHCP Pools screen, click the Add button to create a new DHCP pool. Enter the values shown in the following screen to define the DHCP pool name, network, subnet mask, start and end of the IP address range, DNS server address, domain name, and default gateway router. Click OK when you have entered all the values.

- Why is the starting IP address set to 192.168.1.2 instead of 192.168.1.1?

- In the Deliver Configuration to Router window, review the CLI commands that were generated by the SDM. These are the commands that are delivered to the router to configure DHCP. The commands can also be manually entered from the CLI to accomplish the same task, which you will do in Task 2 of this lab. Do not check the box Save running config to router’s startup config. Click Deliver to finish configuring the router.

Note: By default, the commands that you just generated only update the running configuration file when delivered. When finished configuring the router for DHCP with SDM, you will configure DHCP using the CLI. When you restart the router, you want it to revert back to the configuration that you saved in Step 2.

- What is the purpose of the last two commands in this configuration?

- After the commands are delivered, the final DHCP screen showing the details of the DHCP pool is displayed.

- Choose File > Exit from the SDM main menu to end the SDM session. Click Yes to confirm exiting SDM.

Step 8: Test the DHCP pool configuration with SDM.

- On the customer host H2, open a command prompt, and issue the ipconfig command.

- What IP address is issued to H2?

c. From host H1, ping the default gateway (the router Ethernet interface). Does the ping succeed?

Troubleshoot as necessary, and do not proceed until the ping is successful.

Task 2: Configure and Verify DHCP Using the CLI

Step 1: Restart the Customer router to remove the DHCP commands added by SDM.

- Because you did not save the DHCP configuration created using SDM to NVRAM, restarting the router restores the basic configuration created in Task 1, Step 2. On the Customer router, issue the reload command.

- When prompted to save the configuration, respond with no.

- When prompted with Proceed with reload? [confirm], press Enter.

- Press Enter at the Press RETURN to get started! prompt. You should now see the Customer> prompt.

Step 2: Check the host DHCP client H2 IP configuration.

- Open a command prompt window on H2 and issue the ipconfig /release and ipconfig /renew commands. Because there is no DHCP server currently configured, it may take a while to timeout.

- At the command prompt, now issue the ipconfig command. What is the IP address and subnet mask for H2?

Step 3: Configure the DHCP server excluded addresses on the Customer router.

To prevent certain addresses from being assigned they must be excluded from the pool. This includes the

IP address of the router Fast Ethernet 0/0 interface (the default gateway). In this lab, also exclude addresses from 192.168.1.101 through 192.168.1.254 to reserve them for other purposes, such as servers and printers, which need to have a fixed IP address.

- To exclude addresses, issue the ip dhcp excluded-address command.

Customer (config) #ip dhcp excluded-address 192.168.1.1 Customer(config)#ip dhcp excluded-address 192.168.1.101 192.168.1.254

- Why do you want to exclude addresses before the DHCP pool is even created?

Step 4: Configure the DHCP pool.

On the Customer router, configure a DHCP pool for the internal clients.

Customer (config) #ip dhcp pool INTERNAL

Customer(dhcp-config)#network 192.168.1.0 255.255.255.0 Customer (dhcp-config) #domain-name abc-widgets. inc Customer (dhcp-config) #default-router 192.168.1.1 Customer (dhcp-config) #dns-server 192.168.1.200

Step 5: Test the DHCP pool for H2.

- On H2, open a command prompt and issue the ipconfig /release and ipconfig /renew commands.

- On H2, issue the ipconfig /all command.

- What IP address is issued to H2?

What is the subnet mask of H2?	.

- What is the default gateway of H2?

- What is the connection-specific DNS suffix (domain name) of host H2?

- What is the DHCP server IP address?

- What is the DNS server IP address?

- What is the MAC address of H2?

- From H2, ping the default gateway (the router Ethernet interface). Does the ping succeed? Troubleshoot as necessary, and do not proceed until the ping is successful.

Step 6: Test the DHCP pool for H1.

- On H1, choose Start > Settings > Control Panel > Network Connections > Local Area Connection and change the IP configuration from static to dynamic to make H1 a DHCP client like host H2. Click the Properties button, and then click Internet Protocol (TCP/IP) Properties. Select Obtain an IP address automatically and Obtain a DNS server address automatically. Click OK to exit the configuration window.

- Open a command prompt on H1 and issue the ipconfig /release and ipconfig /renew commands. Because there is no DHCP server currently configured, it may take a while to timeout.

- At the command prompt, now issue the ipconfig command.

- What IP address is issued to H1?

Step 7: Display the DHCP binding on the Customer router.

- To see the IP address and host hardware (MAC) address combination assigned by the DHCP server, issue the show ip dhcp binding command on the Customer router.

Customer#show ip dhcp binding

IP address	Client-ID/	Lease expiration	Type

Hardware address

192.168.1.2 0100.0bdb.04a5.cd Feb 22 2008 11:19 AM	Automatic

192.168.1.3 0100.07e9.63ce.53 Feb 22 2008 11:27 AM	Automatic

- Do the hardware addresses displayed match those recorded for hosts H1 and H2 in Task 1, Step 1? ______

- On the Customer router, display the characteristics of the DHCP pool using the show ip dhcp pool command.

Customer#show ip dhcp pool Pool INTERNAL :

Utilization mark (high/low)	: 100 / 0

Subnet size (first/next)	: 0 / 0

Total addresses	: 254

Leased addresses	: 2

Pending event	: none

1 subnet is currently in the pool :

Current index	IP address range	Leased addresses

192.168.1.4	192.168.1.1 - 192.168.1.254	2

- How many addresses have been leased? ______

- In the output from the command, what do you think Current Index means?

Step 8: Reflection

- What are some advantages and disadvantages of using DHCP?

- What are some advantages and disadvantages of using SDM to configure DHCP on a router as compared to the CLI?

| Router Interface Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 800 (806) | Ethernet 0 (E0) | Ethernet 1 (E1) |  |  |
| 1600 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (S0) | Serial 1 (S1) |
| 1700 | Fast Ethernet 0 (FA0) | Fast Ethernet 1 (FA1) | Serial 0 (S0) | Serial 1 (S1) |
| 1800 | Fast Ethernet 0/0 (FA0/0) | Fast Ethernet 0/1 (FA0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2500 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (S0) | Serial 1 (S1) |
| 2600 | Fast Ethernet 0/0 (FA0/0) | Fast Ethernet 0/1 (FA0/1) | Serial 0/0 (S0/0) | Serial 0/1 (S0/1) |
| Note: To find out exactly how the router is configured, look at the interfaces. The interface identifies the type of router and how many interfaces the router has. There is no way to effectively list all combinations of configurations for each router class. What is provided are the identifiers for the possible combinations of interfaces in the device. This interface chart does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The information in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface. |  |  |  |  |