| CCNA Discovery Working at a Small-to-Medium Business or ISP |  |
| --- | --- |

Lab 9.2.4 Troubleshooting LAN Connectivity

| Device | Host Name | Interface | IP Address | Subnet Mask | Default Gateway | Switch Port |
| --- | --- | --- | --- | --- | --- | --- |
| R1 | R1 | Fast Ethernet 0/0 | 192.168.1.1 | 255.255.255.0 | N/A | Fast Ethernet 0/2 |
| S1 | S1 | VLAN 1 | 192.168.1.99 | 255.255.255.0 | 192.168.1.1 | N/A |
| H1 | H1 | NIC | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 | Fast Ethernet 0/1 |
| H2 | H2 | NIC | 192.168.1.22 | 255.255.255.0 | 192.168.1.1 | N/A |
| Hub | Hub | 1 | N/A | N/A | N/A | Fast Ethernet 0/3 |

Objectives:

- Build a simple, switched network and verify connectivity.

- Troubleshoot LAN connectivity using the LEDs and show commands to find link problems and duplex and speed mismatches.

Background / Preparation

LAN troubleshooting usually centers on switches, because the majority of LAN users connect to the network via switch ports. Duplex and speed mismatches are more common on switches than on routers. Many

devices are set to auto-negotiate speed and duplex settings. If one device on a link is configured to auto- negotiate and the other side is manually configured with speed and duplex settings, mismatches may occur, leading to collisions and dropped packets.

In this lab, you build a small, switched network with a router and a hub, in addition to workstations. You will alter the speed and duplex settings of device interfaces and observe the effects on link lights and interface status.

Set up a network similar to the one in the topology diagram. Any router that meets the interface requirements displayed in that diagram—such as 800, 1600, 1700, 1800, 2500, or 2600 routers, or a combination of these—can be used. See the Router Interface Summary table at the end of the lab to determine which interface identifiers to use based on the equipment in the lab. Depending on the model of the router and switch, output may vary from what is shown in this lab.

Required Resources

The following resources are required:

- One 1841 router or other router with a Fast Ethernet interface

- One 2960 switch or comparable switch with Fast Ethernet interfaces

- One hub with Ethernet interfaces

- Two Windows XP computers

- Three straight-through Category 5 Ethernet cables

- One crossover Category 5 Ethernet cable (optional if hub has an uplink port)

- One console cable

- Access to the command prompts for each host

- Access to the network TCP/IP configuration for each host

From the host computer, start a HyperTerminal session to the router and switch.

Note: Make sure that the routers and switches have been erased and have no startup configurations. Instructions for erasing the switch and router are provided in the Lab Manual, located on Academy Connection in the Tools section. Check with the instructor if you are unsure of how to do this.

Task 1: Build the Network and Configure Devices

Step 1: Configure basic information on the router and switch.

- Build and configure the network according to the topology diagram and device configuration table. Configure basic settings on router R1. If necessary, see Lab 5.3.5, “Configuring Basic Router Settings with the Cisco IOS CLI,” for instructions on setting the host name, passwords, and interface addresses.

- Configure the basic settings on switch S1 to include the host name, passwords, and VLAN 1 IP address. If necessary, see Lab 5.5.4, “Configuring the Cisco 2960 Switch,” for instructions on configuring the switch settings.

- Save the running configuration on R1 and S1 using the copy running-config startup-config command from privileged EXEC mode.

- Connect the hub to switch S1 using a regular port on the hub and a crossover cable or using the hub uplink port (if present) and a straight-through cable.

Step 2: Configure the hosts.

Configure H1 and H2 with an IP address, subnet mask, and default gateway, according to the device configuration table.

Task 2: Verify Cabling, Interface LEDs, and Link Speed Step 1: Visually inspect the network connections.

- After cabling the network devices, verify the connections. Attention to detail now minimizes the time required to troubleshoot network connectivity issues later.

- Are all cables and terminations in good condition? 	
Step 2: Visually inspect the interface link LEDs.

a. What is the color of the link light for the switch port that H1 is attached to?

b. What is the color of the link light on the H1 NIC?

Step 3: View the link speed for host H1 with local area connections.

- On H1, choose Start > Settings > Control Panel > Network Connections > Local Area Connection.

- What is the connection speed?

Task 3: Verify Switch Interface Information Step 1: Verify interface status.

a. From the HyperTerminal session on S1, use the show ip interface brief command to see the status summary of all interfaces.

S1#show ip interface brief

| Interface | IP-Address | OK? Method Status |  | Protocol |
| --- | --- | --- | --- | --- |
| Vlan1 | 192.168.1.99 | YES manual | up | up |
| FastEthernet0/1 | unassigned | YES unset | up | up |
| FastEthernet0/2 | unassigned | YES unset | up | up |
| FastEthernet0/3 | unassigned | YES unset | up | up |
| FastEthernet0/4 | unassigned | YES unset | down | down |

FastEthernet0/5 unassigned	YES unset down	down

b. Which interfaces have a status of up and a protocol that is up?

Step 2: Verify end-to-end connectivity.

- On H1, open a Command Prompt window by choosing Start > Run and typing cmd. Alternatively,

you can choose Start > All programs > Accessories > Command Prompt.

- Use the ping command to test end-to-end connectivity. Ping from H1 to the default gateway. C:\>ping 192.168.1.1

- Ping from host H1 to host H2.

C:\>ping 192.168.1.22

Note: If the pings are not successful, troubleshoot the router and host configurations and connections.

Step 3: Verify interface status and settings.

To view the speed and duplex settings on a port and whether manual or auto-negotiation features were used,

use the show interface port status command.

a. Display the status for port numbers Fast Ethernet 0/1 and Fast Ethernet 0/3.

| S1#sh | interfaces | FastEthernet 0/1 | status |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Port | Name | Status | Vlan | Duplex | Speed | Type |
| Fa0/1 |  | connected | 1 | a-full | a-100 | 10/100BaseTX |
| S1#sh | int f0/3 status |  |  |  |  |  |
| Port | Name | Status | Vlan | Duplex | Speed | Type |
| Fa0/3 |  | connected | 1 | a-half | a-10 | 10/100BaseTX |

- What is the duplex and speed for port Fast Ethernet 0/1?

- What does the “a-” at the beginning of “full” and “100” mean?

- What is the interface type?

- What is the duplex and speed for port Fast Ethernet 0/3?

- Why is the duplex and speed for Fast Ethernet 0/3 different than Fast Ethernet 0/1?

Step 4: View interface error statistics.

- To get a quick view of switch port error statistics, use the show interface port counters errors command.

S1#show int f 0/1 counters errors

Port Align-Err	FCS-Err Xmit-Err	Rcv-Err UnderSize

Fa0/1	0	0	0	0	0

Port	Single-Col Multi-Col Late-Col Excess-Col Carri-Sen Runts Giants

Fa0/1	0	0	0	0	0	0	0

- Are there any errors or collisions for Fast Ethernet 0/1?

- Repeat the command for ports Fast Ethernet 0/2 and Fast Ethernet 0/3.

Task 4: Change Duplex Settings

Step 1: Set the duplex setting to full.

- Change the duplex setting on Fast Ethernet 0/3 to force it to operate at full duplex.

S1 (config) #interface FastEthernet 0/3 S1 (config-if) #duplex full

S1 (config-if) #end

S1#

- What is the result of setting the port Fast Ethernet 0/3 duplex to full?

- Issue the show ip interface brief command. What is the status and protocol for interface 0/3?

- Why did this happen?

Step 2: Set the duplex setting to half duplex.

- Change the duplex setting on Fast Ethernet 0/3 to force it to operate at half duplex.

S1 (config) #interface FastEthernet 0/3 S1 (config-if) #duplex half

S1 (config-if) #end

S1#

- What is the result of setting the port Fast Ethernet 0/3 duplex to half?

- Issue the show ip interface brief command again. What is the status and protocol for interface Fast Ethernet 0/3?

- Why did this happen?

Step 3: Set the duplex setting to auto-negotiate.

- Change the duplex setting on Fast Ethernet 0/3 back to auto-negotiate.

S1 (config) #interface FastEthernet 0/3 S1 (config-if) #duplex auto

S1 (config-if) #end

S1#

- What is the result of setting the port Fast Ethernet 0/3 duplex back to auto?

Task 5: Change Speed Settings

Step 1: Set the speed to 100 Mbps.

- Change the speed setting on Fast Ethernet 0/3 to 100 Mbps.

S1 (config) #interface FastEthernet 0/3 S1 (config-if) #speed 100

S1 (config-if) #end

S1#

- What is the result of setting the speed to 100?

- Issue the show ip interface brief command. What is the status and protocol for interface Fast Ethernet 0/3?

- Why did this happen?

Step 2: Set the speed setting to auto-negotiate.

- Change the duplex setting on Fast Ethernet 0/3 back to auto-negotiate.

S1 (config) #interface FastEthernet 0/3 S1 (config-if) #speed auto

S1 (config-if) #end

S1#

- What is the result of setting the port Fast Ethernet 0/3 speed back to auto?

Task 6: Set Both Duplex and Speed Settings

Step 1: Set the duplex and speed settings for Fast Ethernet 0/1 to full and 100 Mbps.

It is sometimes necessary to set the speed and duplex of a port to ensure that it operates in a particular mode. To force Fast Ethernet port 0/1 to operate at full duplex and 100 Mbps, issue the following commands.

S1 (config) #interface FastEthernet 0/1 S1 (config-if) #duplex full

S1 (config-if) #speed 100

S1 (config-if) #end

S1#

Step 2: Verify the new settings.

a. When a port is in the default state of auto duplex and auto speed, duplex and speed commands do not appear in the running configuration for the interface. When the duplex and speed are set to force the port to operate in a particular mode, the commands used are displayed. Use the show run interface command to view only the portion of the running configuration that is associated with Fast Ethernet 0/1.

S1(config)#show run interface FastEthernet 0/1 Building configuration...

Current configuration : 57 bytes

!

interface FastEthernet0/1 speed 100

duplex full

end

b. Are there any console messages regarding the link status of Fast Ethernet 0/1? Why?

Task 7: Check Settings and Characteristics of Neighboring Devices and Interfaces

Step 1: Check the characteristics of the neighbor attached to switch port Fast Ethernet 0/2. a. Issue the show cdp neighbors command for the S1 Fast Ethernet 0/2 port.

S1#show cdp neighbors FastEthernet 0/2 detail

Device ID: R1

Entry address(es):

IP address: 192.168.2.1

Platform: Cisco 1841, Capabilities: Router Switch IGMP

Interface: FastEthernet0/2, Port ID (outgoing port): FastEthernet0/1 Holdtime : 145 sec

Version :

Cisco IOS Software, 1841 Software (C1841-ADVIPSERVICESK9-M), Version 12.4 (10b),

RELEASE SOFTWARE (fc3)

Technical Support: http://www.cisco.com/techsupport

Copyright (c) 1986-2007 by Cisco Systems, Inc.

Compiled Fri 19-Jan-07 15:15 by prod _rel _team

advertisement version: 2 VTP Management Domain: '' Duplex: full

Management address(es):

- What is the name and platform of the attached device?

- What is the Cisco IOS version?

- What is the duplex setting for the attached port?

- Issue the show cdp neighbors command for S1 Fast Ethernet 0/3.

S1#sh cdp neig f0/3

Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge

S - Switch, H - Host, I - IGMP, r - Repeater, P - Phone Device ID	Local Intrfce Holdtme	Capability	Platform Port ID

- Why is there no information shown for the attached device?

Task 8: Change Router Duplex Settings

Step 1: Set the duplex setting for R1 Fast Ethernet 0/0 to half duplex.

- To force R1 Fast Ethernet port 0/0 to operate at half duplex, issue the following commands.

R1 (config) #interface FastEthernet 0/0 R1 (config-if) #duplex half

R1 (config-if) #end

- Issue the show ip interface brief command on R1.

- What is the status of Fast Ethernet 0/0?

- Issue the show ip interface brief command on S1.

- What is the status of Fast Ethernet 0/2 (the port to which R1 is attached)?

- Can you ping the switch VLAN 1 address (192.168.1.99)? ____

Why?

Task 9: Reflection

When LAN connectivity problems exist, always check link lights first and then check the cabling and terminations. Verify that interfaces are not shutdown. Verify that ports are set to auto-negotiate, if possible. If a device connected to a port cannot auto-negotiate or connectivity problems exist, forcing the port to operate at the specific duplex and speed of the attached device may be required. Check interface errors to determine if there is a problem with the physical interface itself. Always check both ends of the connection, if possible.

| Router Interface Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 800 (806) | Ethernet 0 (E0) | Ethernet 1 (E1) |  |  |
| 1600 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (S0) | Serial 1 (S1) |
| 1700 | Fast Ethernet 0 (FA0) | Fast Ethernet 1 (FA1) | Serial 0 (S0) | Serial 1 (S1) |
| 1800 | Fast Ethernet 0/0 (FA0/0) | Fast Ethernet 0/1 (FA0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2500 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (S0) | Serial 1 (S1) |
| 2600 | Fast Ethernet 0/0 (FA0/0) | Fast Ethernet 0/1 (FA0/1) | Serial 0/0 (S0/0) | Serial 0/1 (S0/1) |
| Note: To find out exactly how the router is configured, look at the interfaces. The interface identifies the type of router and how many interfaces the router has. There is no way to effectively list all combinations of configurations for each router class. What is provided are the identifiers for the possible combinations of interfaces in the device. This interface chart does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The information in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface. |  |  |  |  |