| CCNA Discovery Working at a Small-to-Medium Business or ISP |  |
| --- | --- |

Lab 9.2.3 Identifying Cabling and Media Errors

| Device | Host Name | Interface | IP Address | Subnet Mask | Default Gateway | Switch Port |
| --- | --- | --- | --- | --- | --- | --- |
| R1 | R1 | Fa0/0 | 192.168.1.1 | 255.255.255.0 | N/A | N/A |
|  |  | Fa0/1 | 192.168.2.1 | 255.255.255.0 | N/A | Fa0/1 |
| S1 | S1 | VLAN 1 | 192.168.2.99 | 255.255.255.0 | 192.168.2.1 | N/A |
| H1 | H1 | NIC | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 | N/A |
| H2 | H2 | NIC | 192.168.2.22 | 255.255.255.0 | 192.168.2.1 | Fa0/2 |

Objectives

- Identify Ethernet device and cabling connectivity.

- Build a simple, routed multi-LAN network and verify connectivity.

- Use the show interfaces and show ip interface Cisco IOS commands to observe the symptoms when using the wrong cable.

Background / Preparation

In this lab, you build a simple, multi-LAN routed Ethernet network using different types of cables to connect hosts and networking devices, while observing symptoms of connectivity problems.

Common cable or media issues that can cause connectivity problems include:

- Loose cable or too much tension on the cable – If all the pins cannot make a good connection, the circuit is down.

- Incorrect termination – Ensure that the correct standard is followed, and that all pins are correctly terminated in the connector.

- Damaged serial interface connector – Pins on the interface connection are bent or missing.

- Break or short in the cable – If there are problems along the circuit, the interface cannot sense the correct signals.

- Incorrect cable used – Interchanging straight-through, crossover, and rollover cables can produce unpredictable results and cause lack of connectivity.

Set up a network similar to the one in the topology diagram. Any router that meets the interface requirements displayed in that diagram—such as 800, 1600, 1700, 1800, 2500, or 2600 routers, or a combination of these—can be used. See the Router Interface Summary table at the end of the lab to determine which interface identifiers to use based on the equipment in the lab. Depending on the model of the router, output may vary from what is shown in this lab.

Required Resources

The following resources are required:

- One 1841 router or other router with two Fast Ethernet interfaces

- One 2960 switch or comparable switch with Fast Ethernet interfaces

- Two Windows XP computers

- Two straight-through Category 5 Ethernet cables

- One crossover Category 5 Ethernet cable

- One RJ-45 rollover console cable

- Access to the command prompts for each host

- Access to the network TCP/IP configuration host From a host computer, start a HyperTerminal session to the router and switch.

Note: Make sure that the routers and switches have been erased and have no startup configurations. Instructions for erasing the switch and router are provided in the Lab Manual, located on Academy Connection in the Tools section. Check with the instructor if you are unsure of how to do this.

Task 1: Review Ethernet Device Cabling

Step 1: Complete the Ethernet device interconnection table.

Enter the type of cable required to interconnect the devices listed. Use C for crossover and S for straight- through.

|  | Hub | Switch | Router | Workstation |
| --- | --- | --- | --- | --- |
| Hub |  |  |  |  |
| Switch |  |  |  |  |
| Router |  |  |  |  |
| Workstation |  |  |  |  |

Step 2: Analyze the cabling requirements for this lab.

- What cable type is needed to connect host H1 to router R1, and why?

- What cable type is needed to connect host H1, H2, and router R1 to switch S1, and why?

Task 2: Build the Network and Configure Devices

Step 1: Configure basic information on the router and switch.

- Build and configure the network according to the topology diagram and device configuration table. Configure basic settings on R1. If necessary, see Lab 5.3.5, “Configuring Basic Router Settings with the Cisco IOS CLI,” for instructions on setting the host name, passwords, and interface addresses.

- Configure the basic settings on S1 to include the host name, passwords, and VLAN 1 IP address. If necessary, see Lab 5.5.4, “Configuring the Cisco 2960 Switch,” for instructions on configuring the switch settings.

- Save the running configuration on R1 and S1 using the copy running-config startup-config command from privileged EXEC mode.

Step 2: Configure the hosts.

Configure H1 and H2 with an IP address, subnet mask, and default gateway, according to the device configuration table.

Task 3: Verify Cabling and Interface Link LEDs Step 1: Visually inspect the network connections.

- After cabling the network devices, verify the connections. Attention to detail now minimizes the time required to troubleshoot network connectivity issues later.

- Are all cables and terminations in good condition?	
Step 2: Visually inspect the interface link LEDs.

- What is the color of the link light for the switch port that H2 is attached to?

- What is the color of the link light on the H1 NIC? 	
Task 4: Verify Interface Status and Connectivity

Step 1: Verify interface status using the show ip interface brief command.

- From the HyperTerminal session on R1, use the show ip interface brief command to view a summary of the device interfaces. This command may be abbreviated to sh ip int br.

R1#show ip interface brief

Interface	IP-Address	OK? Method Status	Protocol

FastEthernet0/0 192.168.1.1	YES manual up	up

FastEthernet0/1 192.168.2.1	YES manual up	up

Serial0/0/0	unassigned	YES NVRAM administratively down down

Serial0/0/1	unassigned	YES NVRAM administratively down down

Vlan1	unassigned	YES NVRAM up	down

- What is the interface and protocol status of Fast Ethernet 0/0 and 0/1?

- What does Status in column 4 show with regard to cabling and keepalives?

- What does Protocol in column 5 refer to?

- Why is the status for Serial0/0/0 shown as administratively down?

- On router R1, enable the Serial0/0/0 interface using the no shutdown command.

R1 (config) #interface s0/0/0

R1 (config-if) #no shutdown

*Mar 1 16:00:02.707: %LINK-3-UPDOWN: Interface Serial0/0/0, changed state to down

- Issue the show ip interface brief command again. What is the status for Serial0/0/0 now, and why?

R1#show ip interface brief

Interface	IP-Address	OK? Method Status	Protocol

FastEthernet0/0 192.168.1.1	YES manual up	up

FastEthernet0/1 192.168.2.1	YES manual up	up

Serial0/0/0	unassigned	YES NVRAM down	down

Serial0/0/1	unassigned	YES NVRAM administratively down down

Vlan1	unassigned	YES NVRAM up	down

- Why is the protocol down for the Serial0/0/0 interface?

Step 2: Verify Fast Ethernet interface status using the show interfaces command.

a. On R1, use the show interfaces command to view detailed information for interface Fast Ethernet 0/0.

R1#show interfaces fastEthernet 0/0

FastEthernet0/0 is up, line protocol is up

Hardware is Gt96k FE, address is 001b.5325.256e (bia 001b.5325.256e) Description: LAN 192.168.1.0/24 Default Gateway

Internet address is 192.168.1.1/24

MTU 1500 bytes, BW 100000 Kbit, DLY 100 usec,

reliability 255/255, txload 1/255, rxload 1/255 Encapsulation ARPA, loopback not set

Keepalive set (10 sec)

Full-duplex, 100Mb/s, 100BaseTX/FX

ARP type: ARPA, ARP Timeout 04:00:00

Last input 00:00:50, output 00:00:07, output hang never Last clearing of "show interface" counters never

Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0 Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec 142 packets input, 20117 bytes

Received 135 broadcasts, 0 runts, 0 giants, 0 throttles 0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored

0 watchdog

0 input packets with dribble condition detected 693 packets output, 70950 bytes, 0 underruns

0 output errors, 0 collisions, 2 interface resets 0 babbles, 0 late collision, 0 deferred

0 lost carrier, 0 no carrier

0 output buffer failures, 0 output buffers swapped out

- What is the status and line protocol of this interface?

- What is the reliability of this interface?

- What is the encapsulation of this interface?

- What are the duplex and speed settings of this interface?

- Are there any runts, giants, input errors, CRC errors, output errors, collisions, or interface resets?

- On R1, use the show interfaces command to view detailed information for interface Fast Ethernet 0/1.

R1#show interfaces fastEthernet 0/1

FastEthernet0/1 is up, line protocol is up

Hardware is Gt96k FE, address is 001b.5325.256f (bia 001b.5325.256f) Description: LAN 192.168.2.0/24 Default Gateway

Internet address is 192.168.2.1/24

MTU 1500 bytes, BW 100000 Kbit, DLY 100 usec,

reliability 255/255, txload 1/255, rxload 1/255 Encapsulation ARPA, loopback not set

Keepalive set (10 sec)

Full-duplex, 100Mb/s, 100BaseTX/FX

ARP type: ARPA, ARP Timeout 04:00:00

Last input 00:00:00, output 00:00:03, output hang never Last clearing of "show interface" counters never

Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0 Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 1 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec 5659 packets input, 536086 bytes

Received 5642 broadcasts, 0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored 0 watchdog

0 input packets with dribble condition detected 775 packets output, 68357 bytes, 0 underruns

0 output errors, 0 collisions, 1 interface resets 0 babbles, 0 late collision, 0 deferred

0 lost carrier, 0 no carrier

0 output buffer failures, 0 output buffers swapped out

- Are there any runts, giants, input errors, CRC errors, output errors, collisions, or interface resets?

Step 3: Verify connectivity.

- On host H1, open a Command Prompt window by selecting Start > Run and typing cmd. Alternatively, you can select Start > All programs > Accessories > Command Prompt.

- Ping from H1 to the R1 LAN default gateway. C:\>ping 192.168.1.1

- Use the ping command to test end-to-end connectivity. Ping from host H1 on the R1 192.168.1.0/24 LAN to host H2 on the R1 192.168.2.0/24 LAN.

C:\>ping 192.168.2.22

- Were the pings successful?

Note: If the pings are not successful, troubleshoot the router and host configurations and connections.

Task 5: Observe the Effects of Using Different Cables

Note: The results of this task depend on the type of NIC on the host. If it is a newer NIC, it may be able to auto-detect the transmit (TX) and receive (RX) pairs and adjust accordingly. If this is the case, regardless of whether a straight-through or crossover cable is used, the link lights remain lit on the Fa0/0 interface, and the NIC and the show ip interface brief command shows up/up after a brief adjustment period.

Step 1: Change the cable from host H1 to router R1.

Replace the crossover cable from H1 to the R1 Fa0/0 interface with a straight-through cable. Step 2: Visually inspect the interface link LEDs.

- What is the color of the link light on the R1 interface Fa0/0 that host H1 is attached to?

- What is the color of the link light on the host H1 NIC?	
Step 3: Verify interface status.

- From the HyperTerminal session on R1, use the show ip interface brief command to view a summary of the device interfaces.

R1#show ip interface brief

Interface	IP-Address	OK? Method Status	Protocol

FastEthernet0/0 192.168.1.1	YES manual up	down

FastEthernet0/1 192.168.2.1	YES manual up	up

Serial0/0/0	unassigned	YES NVRAM down	down

Serial0/0/1	unassigned	YES NVRAM administratively down down

Vlan1	unassigned	YES NVRAM up	down

- What is the interface and protocol status of Fast Ethernet 0/0 and 0/1?

- On R1, use the show interfaces fastethernet 0/0 command to view detailed information for each router Fast Ethernet interface.

R1#show interfaces fastEthernet 0/0

FastEthernet0/0 is up, line protocol is down

Hardware is Gt96k FE, address is 001b.5325.256e (bia 001b.5325.256e)

Description: LAN 192.168.1.0/24 Default Gateway Internet address is 192.168.1.1/24

MTU 1500 bytes, BW 100000 Kbit, DLY 100 usec,

reliability 255/255, txload 1/255, rxload 1/255 Encapsulation ARPA, loopback not set

Keepalive set (10 sec)

Auto-duplex, 100Mb/s, 100BaseTX/FX

ARP type: ARPA, ARP Timeout 04:00:00

Last input 00:12:15, output 00:12:19, output hang never Last clearing of "show interface" counters never

Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0 Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec 348 packets input, 42237 bytes

Received 327 broadcasts, 0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored 0 watchdog

0 input packets with dribble condition detected 1022 packets output, 101376 bytes, 0 underruns

0 output errors, 0 collisions, 2 interface resets 0 babbles, 0 late collision, 0 deferred

0 lost carrier, 0 no carrier

0 output buffer failures, 0 output buffers swapped out

d. Are there any runts, giants, input errors, CRC errors, output errors, collisions, or interface resets?

Step 4: Change the cable again from host H1 to router R1.

Replace the cable from H1 to the R1 Fa0/0 interface with a rollover cable.

Step 5: Visually inspect the interface link LEDs.

- What is the color of the link light on the R1 interface Fa0/0 that host H1 is attached to?

- What is the color of the link light on the host H1 NIC?

Step 6: Verify interface status.

- View a summary of the device interfaces. R1#show ip interface brief

Interface	IP-Address	OK? Method Status	Protocol

FastEthernet0/0 192.168.1.1	YES manual up	down

FastEthernet0/1 192.168.2.1	YES manual up	up

Serial0/0/0	unassigned	YES NVRAM down	down

Serial0/0/1	unassigned	YES NVRAM administratively down down

Vlan1	unassigned	YES NVRAM up	down

- What is the interface and protocol status of Fast Ethernet 0/0 and 0/1?

- View detailed information for each router Fast Ethernet interface.

R1#show interfaces fastEthernet 0/0

FastEthernet0/0 is up, line protocol is down

Hardware is Gt96k FE, address is 001b.5325.256e (bia 001b.5325.256e) Description: LAN 192.168.1.0/24 Default Gateway

Internet address is 192.168.1.1/24

MTU 1500 bytes, BW 100000 Kbit, DLY 100 usec,

reliability 255/255, txload 1/255, rxload 1/255 Encapsulation ARPA, loopback not set

Keepalive set (10 sec)

Auto-duplex, 100Mb/s, 100BaseTX/FX

ARP type: ARPA, ARP Timeout 04:00:00

Last input 00:12:15, output 00:12:19, output hang never Last clearing of "show interface" counters never

Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0 Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec 348 packets input, 42237 bytes

Received 327 broadcasts, 0 runts, 0 giants, 0 throttles 0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored

0 watchdog

0 input packets with dribble condition detected 1022 packets output, 101376 bytes, 0 underruns

0 output errors, 0 collisions, 4 interface resets 0 babbles, 0 late collision, 0 deferred

0 lost carrier, 0 no carrier

0 output buffer failures, 0 output buffers swapped out

- Are there any runts, giants, input errors, CRC errors, output errors, collisions, or interface resets?

Step 7: Change the cable from host H2 to switch S1.

Replace the straight-through cable from host H2 to the switch S1 Fa0/2 interface with a crossover cable. Step 8: Visually inspect the interface link LEDs.

- What is the color of the link light on the S1 interface Fa0/2 that host H2 is attached to?

- What is the color of the link light on the host H2 NIC?	
Step 9: Verify interface status.

a. From the HyperTerminal session on S1, use the show ip interface brief to view a summary of the device interfaces.

S1#show ip interface brief

Interface	IP-Address	OK? Method Status	Protocol

Vlan1	192.168.2.99	YES NVRAM up	up

FastEthernet0/1	unassigned	YES unset up	up

FastEthernet0/2	unassigned	YES unset down	down

- What is the interface and protocol status of FastEthernet 0/1 and 0/2?

Note: Depending on the switch model and NIC, the LED may be green and the interface may show as up/up. Some switch ports and NICs will automatically adjust to either a straight-through or crossover.

- On switch S1, use the show interfaces fastethernet 0/0 to view detailed information for that interface.

S1#show interface f 0/2

FastEthernet0/24 is down, line protocol is down (notconnect) Hardware is Fast Ethernet, address is 001d.4635.0c98 (bia 001d. 4635. 0c98)

MTU 1500 bytes, BW 10000 Kbit, DLY 1000 usec,

reliability 255/255, txload 1/255, rxload 1/255 Encapsulation ARPA, loopback not set

Keepalive set (10 sec)

Auto-duplex, Auto-speed, media type is 10/100BaseTX

input flow-control is off, output flow-control is unsupported ARP type: ARPA, ARP Timeout 04:00:00

Last input never, output never, output hang never Last clearing of "show interface" counters never

Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0 Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

0 packets input, 0 bytes, 0 no buffer

Received 0 broadcasts (0 multicast)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored 0 watchdog, 0 multicast, 0 pause input

0 input packets with dribble condition detected 0 packets output, 0 bytes, 0 underruns

0 output errors, 0 collisions, 1 interface resets 0 babbles, 0 late collision, 0 deferred

0 lost carrier, 0 no carrier, 0 PAUSE output

0 output buffer failures, 0 output buffers swapped out

Task 7: Reflection

The note in Task 5 indicated that a modern NIC may be able to sense whether the cable is a straight- through or crossover and adjust accordingly. Why would a NIC not be able to adjust when a rollover cable was used instead of a straight-through or crossover?

| Router Interface Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 800 (806) | Ethernet 0 (E0) | Ethernet 1 (E1) |  |  |
| 1600 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (S0) | Serial 1 (S1) |
| 1700 | Fast Ethernet 0 (FA0) | Fast Ethernet 1 (FA1) | Serial 0 (S0) | Serial 1 (S1) |
| 1800 | Fast Ethernet 0/0 (FA0/0) | Fast Ethernet 0/1 (FA0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2500 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (S0) | Serial 1 (S1) |
| 2600 | Fast Ethernet 0/0 (FA0/0) | Fast Ethernet 0/1 (FA0/1) | Serial 0/0 (S0/0) | Serial 0/1 (S0/1) |
| Note: To find out exactly how the router is configured, look at the interfaces. The interface identifies the type of router and how many interfaces the router has. There is no way to effectively list all combinations of configurations for each router class. What is provided are the identifiers for the possible combinations of interfaces in the device. This interface chart does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The information in the parenthesis is the legal abbreviation that is used in Cisco IOS commands to represent the interface. |  |  |  |  |