# Module 3 – Configuring a Router

# Module overview

Four Diagrams

Diagram 1, Tabular

Upon Completion of this module, the student will be able to perform tasks relating to the following:

- Configure a Router

- Finishing the Configuration

Diagram 2, Tabular

CCNA 640-801 Exam

Implementation and Operation

•  Configure IP addresses, subnet masks, and gateway addresses on routers

and hosts

•  Configure a router for additional administrative functionality

•  Implement a LAN

•  Manage system image and device configuration files

•  Perform an initial configuration on a router

Diagram 3, Tabular

ICND 640-811 Exam

Planning and Designing

•  Design or modify a simple LAN using Cisco products

Implementation and Operation

•  Configure IP addresses, subnet masks, and gateway addresses on routers and

hosts

•  Configure a router for additional administrative functionality

•  Implement a LAN

Diagram 4, Tabular

INTRO 640-821 Exam

Implementation and Operation

•  Establish communication between a terminal device and the router IOS, and

use IOS for system analysis

•  Manipulate system image and device configuration files

•  Perform an initial configuration on a router and save the resultant configuration

file

# Module 3.1: Configure A Router

# Section 3.1.1: CLI Command Modes

Single Diagram

Diagram 1, Tabular/List

Overview of Router Modes

Description - Lists the Router modes,

- 'User EXEC' - uses the prompt 'Router>'

- 'Privileged EXEC' - uses the prompt 'Router#'

- 'Global Configuration' - uses the prompt 'Router(config)#'

The following configuration specific modes are also given

- 'Interface' - uses the prompt 'Router(config-if)#'

- 'Subinterface' - uses the prompt 'Router(config-subif)#'

- 'Controller' - uses the prompt 'Router(config-controller)#'

- 'Map-list' - uses the prompt 'Router(config-map-list)#'

- 'Map-class' - uses the prompt 'Router(config-map-class)#'

- 'Line' - uses the prompt 'Router(config-line)#'

- 'Router' - uses the prompt 'Router(config-router)#'

- 'IPX-router' - uses the prompt 'Router(config-ipx-router)#'

- 'Route-map' - uses the prompt 'Router(config-route-map)#'

# Section 3.1.2: Configuring a Router Name

Single Diagram

Diagram 1, Screenshot

Description - A single open window, displays the following text

"Router(config)# hostname Tokyo

Tokyo(config)# "

The command 'hostname' allows the router name to be set/changed

# Section 3.1.3: Configuring Router Passwords

Single Diagram

Diagram 1, Screen Prompts

Console Password:

Description - the following Prompt/response is displayed

" Router(config)#line console 0

Router(config-line)#password cisco

Router(config-line)#login "

Virtual Terminal Password:

Description - the following Prompt/response is displayed

" Router(config)#line vty 0 4

Router(config-line)#password cisco

Router(config-line)#login "

Enable Password:

Description - the following Prompt/response is displayed

" Router(config)#enable password san-fran "

Perform Password Encryption

Description - the following Prompt/response is displayed

" Router(config)#service password-encryption

Router(config)#enable secret  "

# Section 3.1.4: Examining The Show Commands

Single Diagram

Diagram 1, Screenshot

Description - The responses of the commands 'show version', 'show flash', and 'show users' can be seen.

The 'show version' command gives the following information

- Displays information about the currently loaded software version along with hardware and

device information.

The 'show flash' command gives the following information

- Displays information about flash memory and whot IOS files are stored there.

The 'show users' command gives the following information

- Displays all users who are connected to the router.

# Section 3.1.5: Configuring a Serial Interface

Single Diagram

Diagram 1, Screen Prompts

Interface Configuration Commands

In the following commands, the type argument includes serial, Ethernet, fastethernet, token ring, and others. The following is displayed

" Router(config)#interface type port

Router(config)#interface type slot/port "

The following command is used to administratively turn off the interface. The following is displayed.

" Router(config-if)#shutdown  "

The following command is used to turn on an interface which has been shutdown. The following is displayed.

" Router(config-if)#no shutdown  "

The following command is used to quit the current interface configuration mode. The followinf is displayed

" Router(config-if)#exit  "

# Section 3.1.6: Making Configuration Changes

Single Diagrams

Diagram 1, Flowchart

Making changes to a Router Configuration

State 1 - Make changes in configuration modes, go to State 2.

State 2 - Examine Results "Router#show running-config", Go to State 3.

State 3 - Decision box, Intended Results (Yes/No). 'Yes' go to State 4, 'No' go to

State6

State 4 - Save changes to backup

" Router# copy running-config startup-config

Router# copy running-config tftp  "

Go to State 5.

State 5 - Examine backup file - "Router#show startup-config"

State 6 - Remove changes (execute the following commands):

"Router(config)#no…

Router#copy startup-config running-config

Router#copy tftp running-config

Router#erase startup-config

Router#reload   "

# Section 3.1.7: Configuring an Ethernet Interface

Single Diagram

Diagram 1, Prompt/Response

The following text/commands are displayed

"Router(config)#interface e0

Router(config-if)#ip address 183.8.126.2 255.255.255.128

Router(config-if)#no shutdown   "

# Module 3.2: Finishing The Configuration

# Section 3.2.1: Controlling Remote Access Rights

Single Diagram

Diagram 1, List

Importance of Configuration Standards

- A standard is a set of rules or procedures that are either widely used or

officially specified

- A requirement of configuration standards brings order to the topology and

function of the network.

# Section 3.2.2: Interface Descriptions

Single Diagram

Diagram 1, Relational and Prompt/Response

Configuring Interface Descriptions

Description - displays a router connected to a console which controls the interface Description

The following Prompt/Response is displayed

" Tokyo(config)#interface e 0

Tokyo(config)#description Engineering LAN, Bldg. 18  "

# Section 3.2.3: Configuring An Interface Description

Single Diagram

Diagram 1, Prompt/Response

Interface Description

Procedure 1:

" LAB_A>enable

Password:

LAB_A>#configure terminal

Enter configuration commands, one per line. End with 'CNTL-Z'.

LAB_A(config)#interface ethernet 0

Procedure 2:

" interface Ethernet0

Description LAN Engineering, Bldg. 2

ip address 192.5.5.1 255.255.255.0

no ip directed-broadcast   "

# Section 3.2.4: Login Banners

Single Diagram

Diagram 1, Prompt/Response

Login Banner Displays

The following prompt/Responses are shown

"LAB_A con0 is now available

Press RETURN to get started.

This is a secure system. Authorised Access Only!!!

User Access Verification

Password:

LAB_A>enable

Password:

LAB_A#   "

# Section 3.2.5: Configuring Message-of-the-day (MOTD)

Single Diagram

Diagram 1, Relational

Description - displays a router connected to a console, the console has the following text visible:-

" Tokyo(config)#banner motd  # You have entered a secure system, Authorised Access ONLY!  # "

# Section 3.2.6: Host Name Resolution

Single Diagram

Diagram 1, Prompt/Response

The following is an example of the configuration of a host table on a router.

"Router(config) # ip host Auckland 172.16.32.1

Router(config) # ip host Beirut 192.168.53.1

Router(config) # ip host Capetown 192.168.89.1 
 Router(config) # ip host Denver 10.202.8.1  "

# Section 3.2.7: Configuring Host Tables

Two Diagrams

Diagram 1, Prompt/Response

The IP addresses are the directly attached interfaces on each of the routers in the following network.

"  Lab_A#show hosts

Default domain is not set

Name/Address lookup used domain service

Name servers are

Host	     	 Flags		Age	Type	Address(es)

LAB_A  	(perm, OK)	**	IP	192.5.5.1 205.7.5.1 201.100.11.1

LAB_B  	(perm, OK)	**	IP	219.17.100.2 199.6.13.1 201.100.11.2

LAB_C  	(perm, OK)	**	IP	223.8.151.1 204.204.7.1 199.6.13.2

LAB_D 	(perm, OK)	**	IP	210.93.105.1 204.204.7.2

LAB_E    	(perm, OK)	**	IP	210.93.105.2

Diagram 2, Tabular

Description - Matches the router name, type and ports

Router: 'Lab_A', Type '2514', E0 (192.5.5.1), E1(205.7.5.1),S0(201.100.11.1),S1(--)

Router: 'Lab_B', Type '2501', E0 (219.17.100.1), E1(--),S0(199.6.13.1),S1(201.100.11.2)

Router: 'Lab_C', Type '2501', E0 (223.8.151.1), E1(--),S0(204.204.7.1),S1(199.6.13.2)

Router: 'Lab_D', Type '2501', E0 (210.93.105.1), E1(--),S0(--),S1(204.204.7.2)

Router: 'Lab_E', Type '2501', E0 (210.93.105.2), E1(--),S0(--),S1(--)

# Section 3.2.8: Configuration Backup and Documentation

Single Diagram

Diagram 1, Relational

Description - Displays a router along with the three common forms of backup - 'TFTP server', 'Network Server', 'Disk in a safe place'.

# Section 3.2.9: Backing Up Configuration Files

Two Diagrams

Diagram 1, Prompt/Response

The 'copy running-config tftp' Command

The following text/prompts are displayed

" Router#copy running-config tftp

Remote host []? 131.108.2.155

Name of configuration file to write[Tokyo-config] ?tokyo.2

Write file Tokyo.2 to 131.108.2.155 [confirm] y

Writing Tokyo.2 !!!!!! [OK]   "

Diagram 2, Prompt/Response

The 'copy tftp running-config ' Command

The following text/prompts are displayed

" Router#copy tftprunning-config

Host or network configuration file [host]?

IP address of remote host [255.255.255.255] ? 131.108.2.155

Name of configuration file [Router-config] ?tokyo.2

Configure using Tokyo.2 from 131.108.2.155 [confirm] y

Booting tokyo.2 from 131.108.2.155:!! [OK-874/16000 bytes]

tokyo#  "

# Summary

Single Diagram

Diagram 1, List

Summary

The router has the following modes

- User EXEC mode

- Privileged EXEC mode

- Global configuration mode

- Other configuration modes