- CCNA 3 SWITCHING BASICS AND INTERMEDIATE ROUTING

- MODULE 2 – SINGLE- AREA OSPF

- MODULE OVERVIEW

- Diagram 1. Table

Module 2: Single-area OSPF

Open Completion of this module, the student will be able to perform tasks related to the following:

2.1 Link state routing protocol

2.2 Single-area OSPF concepts

2.3 Single-area OSPF configuration

- Diagram 2. Table

CCNA 640-801 Exam

This module will cover the following objectives for the CCNA 640-801 Exam:

Planning and Designing: Select an appropriate routing protocol based on user requirements

Implementation and Operation: Configure IP address, subnet masks and gateway addresses on routers and hosts. Configure a router for additional administrative functionality. Perform an initial configuration on a routers

- Troubleshooting: Troubleshoot routing protocols

Technology: Evaluate the characteristics of routing protocols

- Diagram 3 Table

ICND 640-811 Exam

This module with cover the following objectives for the ICND 640-811 exam:
Planning and Designing: Select an appropriate routing protocol based on user requirements

Installation and Operation: Configure routing protocols given user requirements. Configure IP address, subnet masks and gateway addresses on routers and hosts. Configure a router for additional administrative functionality.

Troubleshooting: Troubleshoot routing protocols

Technology: Evaluate the characteristics of routing protocols

[2.1.1]    Overview of Link-state Routing

- Diagram 1 Table

Link-state and distance vector protocols

Protocol: Distance vector

Examples: RIP v1 and RIP v2. Interior gateway routing protocol (IGRP)

Characteristics: Copies routing table to neighbors. Updates frequently

RIP v1 and RIP v2 use hop count as metric. Views the network from the perspective of the neighbors. Slow to converge. Susceptible to routing loops. Easy to configure and administer. Consumes a lot of band width

Protocol: Link-State

Examples: Open shortest path first (OSPF). Intermediate-system to intermediate-system (IS-IS)

Characteristics: Uses shortest path. Updates are event triggered. Sends link-state packets to all network routers. Has common view of network. Fast to converge. Not as susceptible to routing loops. Harder to configure. Requires more memory and processing power than distance vector. Consumes less bandwidth than distance vector

[2.1.2]    Link-State Routing Protocol Features

- Diagram 1 Image

Link-State Routing Protocol

Four routers named P1, P2, P3 and P4. P1 has a serial link with P2 and is labeled Perth 1 network.P2 has a serial link with P3 and is labeled Perth 2 network. P3 is link to P1 and P4 via Ethernet and is labeled Perth 3 network. P4 has a subnet labeled Perth 4 network. All the routers are sending Hello packets around the entire network.

- Diagram 2 Image

Link-State Routing Protocol

Four routers named P1, P2, P3 and P4. P1 has a serial link with P2 and is labeled Perth 1 network.P2 has a serial link with P3 and is labeled Perth 2 network. P3 is link to P1 and P4 via Ethernet and is labeled Perth 3 network. P4 has a subnet labeled Perth 4 network. All the routers are sending Hellopackets around the entire network.

- Neighbors of P4 on Perth 3 network are routers P1 and P3.

- Diagram 3 Image

Link-State Routing Protocol

Four routers named P1, P2, P3 and P4. P1 has a serial link with P2 and is labeled Perth 1 network.P2 has a serial link with P3 and is labeled Perth 2 network. P3 is link to P1 and P4 via Ethernet and is labeled Perth 3 network. P4 has a subnet labeled Perth 4 network. All the routers are sending Hello packets around the entire network.

- Neighbors on Perth 3 network are routers P1, P3 and P4.

- Neighbors on Perth 1 network are routers P1 and P2.

- Neighbors on Perth 2 network are router P2 and P3.

[2.1.3]    How Routing Information is Maintained

- Diagram 1 Image

Link-State Routing

There are two networks named Network 1 and Network 2. Router B is on Network 1 and Router D is on Network 2. Router A joins Network 1 to Network 2. Router C joins Network 1 to Network 2 as well. The four routers form a loop between the two networks. All routers are sending Hello packets around the network.

Router B is labeled with a speech bubble “neighbors A and C on Network 1”. Router D is labeled with a speech bubble “neighbors A and C on Network 2”. Router A is labeled with a speech bubble “neighbors B and C on Network 1. Neighbors C and D on Network 2”. Router C is labeled with a speech bubble “neighbors A and B on Network 1. Also neighbors A and D on Network 2”.

[2.1.4]    Link-State Routing Algorithms

- Diagram 1 Image

Link-State Routing Protocol Algorithms

Four routers joined in a ring labeled A, C, D and B. A network cloud hangs off router A with the address 10.0.0.0. A network cloud hangs of router C with the address 192.168.33.0. A network cloud hangs of router D with the address 192.168.167.0. A network cloud hangs of router B with the address 192.168.66.0. Router B is also connected diagonally across the ring to router C.

The cost between router A and router C is 1. The cost between router C and router D is 1. The cost between router D and router B is 1. The cost between router B and router A is 1. The cost diagonally between router B and router C is 1.

- Diagram 2 Table

Entries in Router A Routing Table

Router: A

Destination: 192.168.66.0.

Next Hop: B

Cost: 1

Router: A

Destination: 192.168.33.0.

Next Hop: C

Cost: 1

Router: A

Destination: 192.168.157.0.

Next Hop: B

Cost: 2

Router: A

Destination: 192.168.157.0.

Next Hop: C

Cost: 2

Router: B

Destination: 10.0.0.0

Next Hop: A

Cost: 1

Router: B

Destination: 192.168.33.0.

Next Hop: C

Cost: 1

Router: B

Destination: 192.168.157.0.

Next Hop: D

Cost: 1

Router: C

Destination: 10.0.0.0

Next Hop: A

Cost: 1

Router: C

Destination: 192.168.66.0.

Next Hop: B

Cost: 1

Router: C

Destination: 192.168.157.0

Next Hop: D

Cost: 1

Router: D

Destination: 10.0.0.0

Next Hop: B

Cost: 2

Router: D

Destination: 10.0.0.0

Next Hop: C

Cost: 2

Router: D

Destination: 192.168.66.0.

Next Hop: B

Cost: 1

Router: D

Destination: 192.168.33.0.

Next Hop: C

Cost: 1

[2.1.5]    Advantages and Disadvantages of Link-State Routing

- Diagram 1 Table

Advantages and Disadvantages of Link-State Routing

Advantages: Fast convergence times as changes are reported immediately by the source affected. It provides robustness against routing loops. Routers know the topology. Link-state packets are sequenced and aged. The link-state database sizes can be minimized with careful network design.

Disadvantages: Significant demands on memory and processing resources. Requires very strict network design. Requires a knowledgeable network administrator. Initial flooding can impede network performance.

[2.1.6]    Compare and Contrast Distance Vector and Link-State Routing

- Diagram 1 Table

Comparison of Distance Vector and Link-State Routing

Distance Vector: Views the network topology from the perspective of a neighbor router. Adds distance vectors from router to router. Has frequent, periodic updates and slow convergence. Passes copies of routing tables to neighbor routers.

Link-State Routing: Gets common view of entire network topology. Calculates the shortest path to other routers. Has event-triggered updates with faster convergence. Passes link-state routing updates to other routers.

[2.2.1]    OSPF Overview

- Diagram 1 Image

OSPF vs RIP Route Selection

This is an animation depicting five routers connected in a ring. The connection bandwidths around the ring are T3/E3, T3/E3, T3/E3, 64K and 64K. This is an OSPF routed network. The animation shows the packets following the highest bandwidth path.

- Diagram 2 Image

OSPF vs RIP Route Selection

This is an animation depicting five routers connected in a ring. The connection bandwidths around the ring are T3/E3, T3/E3, T3/E3, 64K and 64K. This is an RIP routed network. The animation shows the packets following the path with the least hops.

- Diagram 3 Image

Single-area OSPF

Eight routers interconnected in a discrete network. Routers at the centre cloud are labeled area 0 backbone.

- Diagram 4 Image

Large OSPF Network

Four interconnected networks, one network labeled Area 0 backbone. Network area 2 has three routes with addresses 1.1.1.0, 1.1.2.0 and 1.1.3.0. Network area 3 has three routers with the addresses 2.2.1.0, 2.2.2.0 and 2.2.4.0. Network area 1 has three routers with the addresses 3.3.1.0, 3.3.2.0 and 3.3.3.0. Link-state changes are invisible outside areas. Large OSPF networks area hierarchical and divided into multiple areas.

[2.2.2]    OSPF Terminology

- Diagram 1 Image

OSPF Terminology

A complex interconnected network of six routers, depicting network areas segmenting off routers into discrete areas. The links between routers are labeled with costs, such as “cost = 10”. Labels show that routers have adjacency databases, topological databases and routing tables. One router is labeled DR and BDR.

- Diagram 2 Image

OSPF Terminology

- Image is the same as the previous diagram. A link is an interface on a router.

- Diagram 3 Image

OSPF Terminology – Link-State

Image is the same as the previous diagram. A link-state is the status of a link between two routers. Also a router interface and it’s relationship to neighbor routers.

- Diagram 4 Image

Link-State Database

Image is the same as the previous diagram. A link-state database (or topological database) is a list of information about all other routers in an inter network.

- Diagram 5 Image

Area

Image is the same as the previous diagram. An area is a collection of networks and routers that has the same area identification. Each router within an area has the same link-state information. A router within an area is called an internal router.

- Diagram 6 Image

Link Cost

Image is the same as the previous diagram. A cost is the value assigned to a link. Link-state protocols assign a cost to a link based on the bandwidth of the link or transmission speed. This is used instead of hops.

- Diagram 7 Image

OSPF Forwarding Database

Image is the same as the previous diagram. A routing table, this is also called a forwarding database. A routing table is generated with an algorithm is run on a link-state database. The routing table for each router is unique.

- Diagram 8 Image

OSPF Adjacency Database

Image is the same as the previous diagram. An adjacency database is a list of all the neighbor routers to which a router has established bidirectional communication. This is unique to each router.

- Diagram 9 Image

OSPF Designated Routers

Image is the same as the previous diagram. A designated router (DR) and backup designated router (BDR) are routers that are elected by all other routers on the same LAN to represent all the routers. Each network has a DR and BDR.

[2.2.3]    Comparing OSPF with Distance Vector Routing Protocols

- Diagram 1 Image

Distance Vector Concepts

Four routers connected in a ring via serial connections named A, B, C and D. There are arrows between each of the connections showing information transfer in both directions.

This image shows the periodic passing of copies of a routing table to neighboring routers and the communication of distance vectors.

- Diagram 2 Image

Routing Loops

Four routers connected in a ring via serial connections named from the top in a clockwise direction B, A, D and C. Hanging off router A is router E connected via fast Ethernet and hanging off of router E is network 1 which is down. A label between router B and A is indicating network 1 as being unreachable. A label between router C and D indicates alternate route through router A to network 1, distance 3. A label between router D and A indicates alternate route through router A to network 1, distance 4. Alternate routes, slow convergence, inconsistent  routing.

- Diagram 3 Image

Problem: Link-State Update

Four routers connected via serial connection in a ring, labeled clockwise from the top B, A, D and C. The link between router D and C is labeled X-OK with the tag “Network 1 goes down then comes back up”. The link between router C and B has an arrow pointing from C to B and is tagged “slow path update, Network 1 unreachable”.  The link between routers B and A has an arrow point from B to A and is tagged “slow path updates arrive last, Network 1 unreachable”. The link between routers A and D has an arrow pointing from D to A and is tagged “fast path updates first, Network 1 unreachable, Network 1 back up now”.

- A tag showing a simple branched network “which SPF tree to use for routing?”.

- Diagram 4 Table

Comparison of OSPF, Link-State and Distance Vector Routing

Distance Vector: Views the network topology from the perspective of the neighbors. Adds distance vectors from router to router. Has frequent, periodic updates and slow convergence. Passes copies of routing tables to neighboring routers. Uses a flat topology.

Link-State: Gets common view of entire network topology. Calculates the shortest path to other routers. Has event triggered updates with faster convergence. Passes link-state routing updates to other routers. Allow hierarchical design for large internetworks.

[2.2.4]    Shortest Path Algorithm

- Diagram 1 Image and Table

Shortest Path First Algorithm

Seven routers connected via fast Ethernet in a ring, labeled from the top in a clockwise direction C, B, A, G, F, E and D.  The link between C and B has a cost of 1. The link between B and A has a cost of 4. The link between A and G has a cost of 2. The link between G and F has a cost of 2. The link between F and E has a cost of 2. The link between E and D has a cost of 1. The link between D and C has a cost of 4. There is also a link between E and C which bypasses D with a cost of 2.

A

B/4

G/2

B

A/4

C/1

C

B/1

D/4

E/2

D

C/4

E/1

E

C/2

D/1

F/2

F

E/2

G/2

G

A/2

F/2

- Diagram 2 Image and Table

Shortest Path First Tree for Node B

Six routers connected via Ethernet in a ring, labeled from the top in a clockwise direction B, A, G, F, E and C. Connected to router E via Ethernet is router D which has no other connections. The link between routers B and A has a cost of 4. Router A is labeled B/4. The link between routers A and G has a cost of 2. Router G is labeled A/6. The link between routers G and F has a cost of 2. Router F is labeled E/5. The link between routers F and E has a cost of 2. Router E is labeled C/3. The link between routers E and C has a cost of 2. Router C is labeled B/1. The link between routers C and B has a cost of 1. The link between router E and D has a cost of 1. Router D is labeled E/4.

A: B/4, G/2

B: A/4, C/1

C: B/1, D/4, E/2

D: C/4, E/1,

E: C/2, D/1, F/2

F: E/2, G/2

G: A/2, F/2

[2.2.5]    OSPF Network Types

- Diagram 1 Image

OSPF Network Types

Three separate images.

Image 1:Broadcast Multiaccess. A number of routers linked together via one central line.

Image 2 point-to-point: two routers connected via a serial link.

Image 3 NBMA: four routers connected via a mesh with a network cloud in the middle labeled “X.25 frame relay”.

- Diagram 2 Table

OSPF Network Types

Network type: Broadcast multiaccess

Characteristics: Ethernet, tonken ring or FDDI

DR Election?: Yes

Network type: Non-broadcast multiaccess

Characteristics: Frame relay, X.25, SMDS

DR Election?: Yes

Network type: Point-to-point

Characteristics: PPP, HDLC

DR Election?: No

Network type: Point-to-multipoint

Characteristics: Configured by an administrator

DR Election?: No

- Diagram 3 Image

DR and BDR Receive LSAs

A central backbone Ethernet line, one router hanging off labeled DR, one router hanging off labeled BDR and three other routers hanging off which are unlabeled. Arrows show that all routers send updates to and from the DR and BDR routers. Updates are also sent between the DR and BDR routers.

[2.2.6]    OSPF Hello Protocol

- Diagram 1 Table

OSPF Packet Header

| Version | Type |  | Packet Length |
| --- | --- | --- | --- |
| Router ID |  |  |  |
| Area ID |  |  |  |
| Checksum |  | Authentication Type |  |
| Authentication Data |  |  |  |

- Diagram 2 Table

OSPF Hello Header

| Network Mask |  |  |
| --- | --- | --- |
| Hello Interval | Options | Router Priority |
| Dead Interval |  |  |
| Designated Router |  |  |
| Backup designated router |  |  |
| Neighbor Router ID |  |  |
| Neighbor Router ID |  |  |
| Additional Network router ID fields can be added to the end of the header is necessary. |  |  |

[2.2.7]    Steps in the Operation of OSPF

- Diagram 1 Image

Discover Neighbors

Three routers named RTA, RTB and RTC from left to right. RTA is connected to RTB via Ethernet link. RTA’s Ethernet port address is 10.5.0.1. RTB’s Ethernet port address is 10.5.0.2. RTB is connected to RTC via a serial link. RTB’s serial port address is 10.6.0.1. RTC’s serial port address is 10.6.0.2. This network is all in area 0. This image is tagged “router IDs, RTA = 10.5.0.1, RTB = 10.6.0.1, RTC = 10.6.0.2.”

An OSPF router tried to form an adjacency with at least one neighbor with each IP network to which it is connected.

- Diagram 2 Image

Elect DR and BDR on a Multiaccess network

Three network clouds each joined by a router. First router is named RTA, second named RTB and the last router at the far edge of the last network cloud is named RTC. The first router cloud is network 10.4.0.0/16, the second router cloud is 10.5.0.0/16 and the last network cloud is 10.6.0.0./16. Each network cloud is a separate Ethernet broadcast and multicast subnet. RTA and RTB are connected via Ethernet, RTB and RTC are connected via a serial connection. RTA’s Ethernet port address is 10.5.0.1, RTB’s Ethernet port address is 10.5.0.2, RTB’s serial port address is 10.6.0.1, RTC’s serial port address is 10.6.0.2. All routers are engaged in sending Hello packets to elect DR and BDR routers.

- OSPF routers perform DR and BDR elections only on multiaccess IP networks.

- Diagram 3 Image and Table

Selecting the Best Route

Three routers in a row named RTA, RTB and RTC. RTA is connected to a subnet with the address 10.4.0.0/16. The link to this subnet has a cost of 10. RTA is also connected to RTB via Ethernet. The subnet between these two routers is 10.5.0.0/16 and the link has a cost of 10. RTB is connected to RTC via a serial link. The subnet between this link is 10.6.0.0/16 and has a cot of 64.

- The table below refers to router RTB.

Net: 10.4.0.0

Cost: 20

Out Interface: E0

The link-state database is processed by the shortest path first algorithm and the best routes are selected.

- Diagram 4 Image

Maintaining Routing Information

A small network of four routers sending LSU packets. One router labeled R1 tagged “new router”, one router named R2, another router name DR and a final router named R3.

R1 detects a downed link and sends an LSU to the DR. The DR acknowledges the receipt of the LSU.

- Diagram 5 Image

Maintaining Routing Information

A small network of four routers sending LSU packets. One router labeled R1 tagged “new router”, one router named R2, another router name DR and a final router named R3.

After the DR acknowledges the receipt of the LSU, it floods the LSU to all OSPF routers on the network using the multicast address 224.0.0.5. Each router acknowledges receipt of the LSU with and LSAck.

- Diagram 6 Image

Maintaining Routing Information

A small network of four routers sending LSU packets. One router labeled R1 tagged “new router”, one router named R2, another router name DR and a final router named R3.

- If an OSPF router is attached to another network, it floods the LSU to other networks.

- Diagram 7 Image

Maintaining Routing Information

A small network of four routers sending LSU packets. One router labeled R1 tagged “new router”, one router named R2, another router name DR and a final router named R3.

After and OSPF router receives an LSU that includes new information, it uploads its link-state database. It then runs the SPF algorithm with the new information to recalculate the routing table.

[2.3.1]    Configuring OSPF Routing Process

- Diagram 1 Image

Basic OSPF Configuration

Three routers named A, B and C. A is connected to B via Ethernet. A’s E0 port is addressed 10.64.0.1/24. B’s E0 port is addressed 10.64.0.2/24. B is connected to C via serial connection. B’s S0 port is 10.2.1.2/30 and C’s S1 port is addressed 10.2.1.1/30.

Router A’s configuration:

<output omitted>

Interface Ethernet0

IP address 10.64.0.1 255.255.255.0

!

<output omitted>

Router OSPF 1

Network 10.64.0.0 0.0.0.255 area 0

Router B’s configuration:

<output omitted>

Interface Ethernet0

IP address 10.64.0.2 255.255.255.0

!

- Interface serial

IP address 10.2.1.2 255.255.255.252

<output omitted>

Router OSPF 1

Network 10.2.1.0 0.0.0.3 area 0

Network 10.64.0.0 0.0.0.255 area 0

- Diagram 2 Table

Basic OSPF Configuration

Network area command: Address

Description: This can be the network address, subnet or address of the interface. It tells routers which links to listen to advertisements on and which links and networks to advertise.

Network area command: Wildcard mask

Description: This is an inverse mask that is used to determine how an address is read. The mask has wildcard bits where 0 is a match and 1 is not important. For example, 0.0.255.255 indicates a match in the first to bytes. The equivalent subnet mask would be a 16 bit mask of 255.255.0.0. The wildcard mask 0.0.0.0 is used to specify an interface address.

Network area command: Area -ID

Description: This value specifies the areas to be associated with an address. It can be a number or can be similar to an IP address. For a backbone area, the ID must equal 0.

[2.3.2]    Configuring OSPF Loopback Address and Router Priority

- Diagram 1 Image

Configuring a Loopback

! create the loopback 0 interface

Sydney3(config)#interface loopback 0

Sydney3(config-if)#ip address 192.168.31.33 255.255.255.255

Sydney3(config-if)#exit

! remove loopback 0 interface

Sydney3(config)#

01:47:27: %link-5-CHANGED: interface lookback0, changed

State 2 administratively down

- Diagram 2 Table

OSPF Hello Packet

| Network Mask |  |  |
| --- | --- | --- |
| Hello Interval | Options | Router Priority |
| Dead Interval |  |  |
| Designated Router |  |  |
| Backup designated router |  |  |
| Neighbor Router ID |  |  |
| Neighbor Router ID |  |  |
| Additional Network router ID fields can be added to the end of the header is necessary. |  |  |

Hello packets carry information about Hello and dead interval times and router identifications. Routers must agree upon this information to form adjacencies.

- Diagram 3 Image

Setting OSPF Priority

- Sydney1(config)#interface fastethernet 0/0

Sydney1(config-if)#ip OSPF priority 50

Sydney1(config-if)#end

Sydney1#

00:21:57: %sys-5-CONFIG_T: configured from console by console

The Hello packet sent on the fast Ethernet interface will have the router priority field set to 50.

- Diagram 4 Image

Show OSPF Interface Properties

- Sydney1>show ip ospf interface fastethernet 0/0

Fastethernet0/0 is up, line protocol if up

Internet address 192.168.1.1/24, area 0

Process TD 1, router TD 192.168.31.11, network

Type BROADCAST, cost:1 transmit delay is 1 sec,

State DROTHER, priority 50

Designated router (TD) 192.168.31.22, interface

- Address 192.168.1.2

Backup designated router (TD) 192.168.31.33,

- Interface address 192.168.1.3

Timer intervals configured, hello 10, dead 10,

- Wait 40, retransmit 5

Hello due in 00:00:03

Index 1/1, flood queue, length 0

Next 0x0 (0) /0x0 (0)

Last flood scan length is 0, maximum is to

Last flood scan time is 0 msec, maximum is 0

[2.3.3]    Modifying OSPF Cost Metric

- Diagram 1 Table

Cisco IOS Default OSPF Path Costs

Link Type and Bandwidth: 56-kpbs serial link

Cost: 1785

Link Type and Bandwidth: T1 1.544-Mbps serial link

Cost: 64

Link Type and Bandwidth: E1 2.048-Mbps serial link

Cost: 48

Link Type and Bandwidth: 4-Mbps token ring

Cost: 25

Link Type and Bandwidth: 10-Mbps Ethernet

Cost: 10

Link Type and Bandwidth: 16-Mbps token ring

Cost: 6

Link Type and Bandwidth: 100-Mbps fast Ethernet, FDDI

Cost: 1

- Diagram 2 Image

Setting Path Cost Explicitly

- Sydney2(config-if)#ip opsf cost ?

<1-65535> cost

Sydney2(config-if)#ip ospf cost 1

[2.3.4]    Configuring OSPF Authentication

- Diagram 1 Table

OSPF Packet Header

| Version | Type |  | Packet Length |
| --- | --- | --- | --- |
| Router ID |  |  |  |
| Area ID |  |  |  |
| Checksum |  | Authentication Type |  |
| Authentication Data |  |  |  |

- Diagram 2 Image

OSPF Authentication with MD5

- Sydney1(config-if)#ip ospf message-digest-key 1 md5 7 secret

Sydney1(config-if)#exit

Sydney1(config)#router ospf 1

Sydney1(config-router)#area 0 authentication message-digest

Sydney1(config-router)#end

Sydney1#

[2.3.5]    Configuring OSPF Timers

- Diagram 1 Image

Configuring OSPF, Hello and Dead Interval Timers

- Sydney1(config-if)#ip ospf hello-interval 5

Sydney1(config-if)#ip ospf dead-interval 20

- OSPF timers are configured on the interface.

[2.3.6]    OSPF Propagating a Default Route

- Diagram 1 Image and Table

Default Routing Example

Two network clouds, first network cloud is subnet 10.0.0.0 and is labeled company X it has four serially interconnect routers in it. The second cloud is labeled internet and has three serially interconnected routers in it. Router B from company X is connected to router C from the internet the subnet for this serial connection is 192.34.56.0.

Routing table: There is no entry for the destination network. Try router B default route.

- Use if the next hop is not explicitly listed in the routing table.

[2.3.7]    Common OSPF Configuration Issues

- Diagram 1 Table

Common OSPF Configuration Issues

No neighbor

Do the interfaces have the same OSPF timers?

Do the connected interfaces have the same network type?

Are the authentication keys and permissions the same on the interfaces?

Do the other neighbors have duplicate IP addresses?

Is the router interface up?

OSPF routers not shown

Do the interfaces have the correct IP addresses and subnet masks?

Do the network statements have the correct wildcard masks?

Do the network statements put links into the correct area?

[2.3.8]    Verifying the OSPF Configuration

- Diagram 1 Table

Verifying the OSPF Configuration Commands

Command: Show IP protocol

Description: This displays parameters for timers, filters, metrics, networks and other information for the entire router.

Command: Show IP route

Description: This displays the routes known to the router and describes how they were learned. This is one of the best ways to determine connectivity between the local router and the rest of the internetwork.

Command: Show IP OSPF Interface

Description: This verifies that interfaces have been configured in the intended areas. If no loopback address is specified the interface with the highest address is taken as the router ID. Is also gives the timer intervals such as the Hello interval and shows the router adjacencies.

Command: Show IP OSPF

Description: This displays the number of times that the SPF algorithm has been used.  It also shows the link-state update intervals if no topological changes have occurred.

Command: Show IP OSPF Neighbor detail

Description: This displays a detailed list of neighbors, their priorities and their state, such as init, exstart or full.

Command: Show IP OSPF database

Description: This displays the contents of the topological database maintained by the router. It also shows the router ID and the OSPF process ID. A number of database types can be shown when this command is used with keywords. Refer to www.cisco.com for details about the keywords.

- Diagram 2 Table

Cisco IOS Debug and Clear Commands for OSPF Verification

Command: Clear IP Route

Description: Clear all router routes in routing table

Command: Clear IP Route A.B.C.D

Description: Clear route to A.B.C.D in routing table

Command: Debug IP OSPF Events

Description: Report all OSPF events

Command: Debug IP OSPF Adj

Description: Report OSPF adjacency events

- Module Summary

- Diagram 1 Table

Module 2: Summary

Summary:

- Link-state routing protocols collect routing information from all other routers in the network or within a defined area of the network.

- Link-state routing protocols perform the following functions:

  - Respond quickly to network changes

  - Send triggered updates only when a network change has occurred

  - Send periodic updates known as link-state refreshers

  - Use a Hello mechanism to determine the reachablity of neighbors

- OPSF is a link-state routing protocol based on open standards.

- OSPF routing uses the concept of areas. Each router contains a complete database of link-states in a specific area.