TMS320C5402 DSK Help (SPRH075A) Copyright© 1998-1999 Texas Instruments

Peripheral Interface.

J8Pin#	Signal Name	Type		J8Pin#	Signal Name	Type

1	+12V	O		2	+12V	O

3	GND	-		4	GND	-

5	+5V	O		6	+5V	O

7	GND	-		8	GND	-

9	+5V	O		10	+5V	O

11	SPARE (N/C)	-		12	SPARE (N/C)	-

13	SPARE (N/C)	-		14	SPARE (N/C)	-

15	SPARE (N/C)	-		16	SPARE (N/C)	-

17	SPARE (N/C)	-		18	SPARE (N/C)	-

19	+3.3V	O		20	+3.3V	O

21	X_CLKX0	I/O/Z		22	SPARE (N/C)	-

23	X_FSX0	I/O/Z		24	X_DX0	O

25	GND	-		26	GND	-

27	X_CLKR-	I/O/Z		28	SPARE (N/C)	-

29	X_FSR0	I/O/Z		30	X_DR0	I

31	GND	-		32	GND	-

33	X_CLKX1	I/O/Z		34	SPARE (N/C)	-

35	X_FSX1	I/O/Z		36	X_DX1	O

37	GND	-		38	GND	-

39	X_CLKR1	I/O/Z		40	SPARE (N/C)	-

41	X_FSR1	I/O/Z		42	X_DR1	I

43	GND	-		44	GND	-

45	X_TOUT	O		46	SPARE (N/C)	-

47	SPARE (N/C)	-		48	X_INT1#	I

49	X_XF	O		50	X_BIO#	I

51	GND	-		52	GND	-

53	X_INT0#	I		54	X_IACK#	O

55	SPARE (N/C)	O		56	X_IOSTRB#	O

57	X_MSC#	O		58	X_IAQ#	O

59	X_RESET#	O		60	X_DBINT#	O

61	GND	-		62	GND	-

63	X_DNTL1	O		64	X_CNTL0	O

65	X_STAT1	I		66	X_STAT0	I

67	X_INT2#	I		68	X_INT3#	I

69	X_PS#	O		70	X_IS#	O

71	SPARE (N/C)	O		72	SPARE (N/C)	-

73	SPARE (N/C)	O		74	SPARE (N/C)	-

75	DB_DET#	I		76	GND	-

77	GND	-		78	X_CLKOUT	O

79	GND	-		80	GND	I

TMS320C5402 DSK Help (SPRH075A) Copyright© 1998-1999 Texas Instruments

What does n/c mean?

The expansion peripheral interface provides the DSP’s peripheral signals to a daughterboard. This peripheral expansion capability allows serial devices (such as other codecs and communication devices) to be added to the DSK via a daughterboard.

The expansion peripheral interface includes:

·	Six signals for each of the serial ports - The DSP's six McBSP signals for McBSP0 and McBSP1 are available on the expansion peripheral interface.  These signals are buffered on the DSK to support both 5-V and 3.3-V serial devices.  Each McBSP has CLKX, FSX, DX, XLKR, FSR and DR signals.

·	Timer output signal -  The DSP's timer output signal, TOUT, is available on the expansion peripheral interface.

·	Interrupts and interrupt acknowledge -  Four  external DSP interrupts (INT0#, INT1#, INT2#, and INT3#), a DSP-to-daughterboard interrupt (X_DBINT#), and the DSP's interrupt acknowledge (IACK#) are available on the expansion peripheral interface.

·	DSP general-purpose I/O signals -  The DSP's XF and BIO# signals are available on the expansion peripheral connector.

·	Three asynchronous memory control signals.  Asynchronous memory control signals that are from the DSP are available on the expansion memory interface.  The signals include X_PS#, X_IS#, and X_IOSTRB#.

·	Reset signal - The expansion peripheral interface also provides a reset signal that is active low when the board is in the reset state.  This allows the circuitry on the daughterboard to be set to a known state.  The reset signal is asserted for a minimum of 140 milliseconds upon power-up, via a manual reset push button or under software control.  A CPLD register bit (DB_RST) allows DSP software to directly control this reset signal when the DSP is not held in reset.

·	CLKOUT/2 signal for the synchronization clock - The DSP's CLKOUT is divided-by-two and is brought out to the expansion peripheral interface for synchronization needs on daughterboards.

·	Daughterboard detect - Pin 75 on the expansion peripheral connector should be grounded on the daughterboard to indicate that the daughterboard is present.

·	Two miscellaneous signals - The DSP's MSC# and IAQ# signals are also provided on the expansion peripheral interface.

·	Power signals - The expansion peripheral interface also provides ground, 12V, -12V, 5V, and 3.3V power signals to the daughterboard.

TMS320C5402 DSK Help (SPRH075A) Copyright© 1998-1999 Texas Instruments